BIOS-LEVEL USB API 0.05

�����������������������

Bret E Johnson

June 30, 2009

TABLE OF CONTENTS

�����������������

OVERVIEW  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  4

GENERAL NOTES ABOUT USB . . . . . . . . . . . . . . . . . . . . . . .  6

OWNERSHIP REGISTRATION  . . . . . . . . . . . . . . . . . . . . . . .  7

NOTIFICATION CODES  . . . . . . . . . . . . . . . . . . . . . . . . 10

IMMEDIATE, SCHEDULE, AND LARGE TRANSACTIONS . . . . . . . . . . . . . 11

STATUS CODES  . . . . . . . . . . . . . . . . . . . . . . . . . . . 14

CALL BACK ADDRESSES & USER PACKET ID's  . . . . . . . . . . . . . . . 15

INTERACTION BETWEEN MULTIPLE HOST DRIVERS . . . . . . . . . . . . . . 17

IRQ's . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

HUBS  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

DEVICE ENUMERATION  . . . . . . . . . . . . . . . . . . . . . . . . . 23

POWER DISTRIBUTION  . . . . . . . . . . . . . . . . . . . . . . . . . 26

ROOT HUBS AND DEVICE ADDRESS 1  . . . . . . . . . . . . . . . . . . . 29

MEMORY ADDRESSES  . . . . . . . . . . . . . . . . . . . . . . . . . . 31

INT 14h API EXTENSIONS  . . . . . . . . . . . . . . . . . . . . . . . 33

Install Check . . . . . . . . . . . . . . . . . . . . . . . . . . . 33

Transaction Request . . . . . . . . . . . . . . . . . . . . . . . . 34

ERROR CODES . . . . . . . . . . . . . . . . . . . . . . . . . . . . 35

64-BYTE REQUEST STRUCTURE . . . . . . . . . . . . . . . . . . . . . . 37

TRANSACTION REQUEST TYPES . . . . . . . . . . . . . . . . . . . . . . 42

Get Host Software Information . . . . . . . . . . . . . . . . . . . 43

Get Host Hardware Information . . . . . . . . . . . . . . . . . . . 44

Get Host Vendor Information . . . . . . . . . . . . . . . . . . . . 46

Get Host Status . . . . . . . . . . . . . . . . . . . . . . . . . . 47

Host Run  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 48

BIOS-LEVEL USB API 0.05            Page 1 of 168

Host Stop . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 50

Host Reset  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 51

Host Suspend (Global Suspend) . . . . . . . . . . . . . . . . . . . 53

Host Resume (Global Resume) . . . . . . . . . . . . . . . . . . . . 55

Register As Frame Timing Owner  . . . . . . . . . . . . . . . . . . 57

Unregister as Frame Timing Owner  . . . . . . . . . . . . . . . . . 59

Increment Frame Timing Factor (Slow Down Bus) . . . . . . . . . . . 60

Decrement Frame Timing Factor (Speed Up Bus)  . . . . . . . . . . . 62

Change Frame Timing Factor (By a Large Amount)  . . . . . . . . . . 64

Get Device Hub Information  . . . . . . . . . . . . . . . . . . . . 66

Notify Host Driver of New Device Connection . . . . . . . . . . . . 67

Notify Host Driver of Device Disconnection  . . . . . . . . . . . . 69

Send Hub Characteristics to Host Driver . . . . . . . . . . . . . . 71

Enable Device Given Hub Address and Port  . . . . . . . . . . . . . 72

Disable Device Given Hub Address and Port . . . . . . . . . . . . . 74

Reset Device Given Hub Address and Port . . . . . . . . . . . . . . 76

Suspend Device Given Hub Address and Port (Selective Suspend) . . . 78

Resume Device Given Hub Address and Port (Selective Resume) . . . . 80

Power On Device Given Hub Address and Port  . . . . . . . . . . . . 82

Power Off Device Given Hub Address and Port . . . . . . . . . . . . 84

Power Reset Device Given Hub Address and Port . . . . . . . . . . . 86

Get Device Power Information  . . . . . . . . . . . . . . . . . . . 88

Get Hub Power Draw  . . . . . . . . . . . . . . . . . . . . . . . . 89

Power On Device Given Device Address  . . . . . . . . . . . . . . . 90

Power Off Device Given Device Address . . . . . . . . . . . . . . . 92

Power Reset Device Given Device Address . . . . . . . . . . . . . . 94

Get Device Class Information  . . . . . . . . . . . . . . . . . . . 96

Get Device Vendor Information . . . . . . . . . . . . . . . . . . . 97

Get Device Status Information . . . . . . . . . . . . . . . . . . . 98

Enable Device Given Device Address  . . . . . . . . . . . . . . .  100

Disable Device Given Device Address . . . . . . . . . . . . . . .  102

Reset Device Given Device Address . . . . . . . . . . . . . . . .  104

Suspend Device Given Device Address . . . . . . . . . . . . . . .  107

Resume Device Given Device Address (Selective Resume) . . . . . .  109

Configuring Interface . . . . . . . . . . . . . . . . . . . . . .  111

Configuring Interface Complete  . . . . . . . . . . . . . . . . .  113

Reset Device and Set New Configuration  . . . . . . . . . . . . .  114

Find Registered Interface . . . . . . . . . . . . . . . . . . . .  117

Find Unregistered Interface . . . . . . . . . . . . . . . . . . .  119

Register As Interface Owner . . . . . . . . . . . . . . . . . . .  121

Register As Device 0 (New Interface) Owner  . . . . . . . . . . .  123

Unregister As Interface Owner . . . . . . . . . . . . . . . . . .  126

Unregister As Device 0 (New Interface) Owner  . . . . . . . . . .  127

Unregister Current Interface Owner and Don't Look . . . . . . . .  128

Get Alternate Interface Information . . . . . . . . . . . . . . .  130

Get Current DataX Value . . . . . . . . . . . . . . . . . . . . .  131

Increment (Toggle) DataX Value  . . . . . . . . . . . . . . . . .  132

Schedule Isochronous Transaction  . . . . . . . . . . . . . . . .  134

Schedule Periodic Interrupt Transaction . . . . . . . . . . . . .  137

Schedule Control (Setup) Transaction  . . . . . . . . . . . . . .  139

Schedule Bulk Transaction . . . . . . . . . . . . . . . . . . . .  143

BIOS-LEVEL USB API 0.05            Page 2 of 168

Schedule One-Time Interrupt Transaction . . . . . . . . . . . . .  145

Close Transaction Handle  . . . . . . . . . . . . . . . . . . . .  147

Change Periodicity of Periodic Interrupt  . . . . . . . . . . . .  149

Get Status of Packet Transaction  . . . . . . . . . . . . . . . .  150

Large Call Completed  . . . . . . . . . . . . . . . . . . . . . .  154

Beep the Speaker  . . . . . . . . . . . . . . . . . . . . . . . .  155

A New Host Driver Has Been Installed  . . . . . . . . . . . . . .  157

An Existing Host Driver Has Been Removed  . . . . . . . . . . . .  158

InterHost Register As Device 0 Owner  . . . . . . . . . . . . . .  159

InterHost Unregister As Device 0 Owner  . . . . . . . . . . . . .  162

Copy the New Device Ownership Table . . . . . . . . . . . . . . .  163

Put Host In Debug Mode  . . . . . . . . . . . . . . . . . . . . .  165

Single Step Host (Debug Mode) . . . . . . . . . . . . . . . . . .  167

BIOS-LEVEL USB API 0.05            Page 3 of 168

OVERVIEW

��������

This document is a technical description of a BIOS-level USB API

(Application Programming Interface) for an IBM-compatible computer.  The

first actual implementation of this API is with a DOS TSR (Terminate-

but-Stay-Resident) program, called USBUHCI.  Although the first

implementation of this API is both operating system-specific (the OS

being DOS) and USB hardware-specific (the hardware being UHCI), it is in

fact intended to be a general-purpose, BIOS-level API.  The API (or

something very similar to it) could, and in my opinion should, be

implemented natively in the BIOS of all computers.  This would allow USB

devices to be used in virtually any operating environment.

This API is implemented as an extension to the BIOS serial services

interrupt (INT 14h).  The API allows for control, monitoring, and

scheduling of USB host resources, including device attachments and

detachments, frame timing, transaction scheduling, hub control, and

interaction between multiple host drivers.  The API is also designed to

operate almost completely as a background operation.  This allows for

true plug-and-play capabilities, even in single-tasking operating

systems like DOS.

This scope of this API includes control and monitoring of the host

controller(s), the USB root hub(s), and the extension of the bus(ses) to

external devices (including remote hubs).  The API specifically does not

cover the multitude of different types of devices that may attach to the

bus(ses), other than hubs.  This API will typically be implemented as a

host driver, and there will then be one or more device/interface drivers

for each bus-attached device/interface that the PC user wishes to

use/control/monitor.  A remote hub is a special type of USB device, in

that it is both a bus-attached device and also the mechanism that allows

the physical and logical extension of the bus to other devices.  This

API includes functions that allow the host driver(s) and hub driver(s)

to communicate and provide that logical Bus extension.

There can be multiple USB host controllers installed on a single

computer.  The API addresses that situation, and allows up to 16 host

controllers to be functioning on the same computer at the same time.

Host drivers can be installed in several different ways: natively (and

permanently) in the BIOS, with permanent (non-removable) host drivers

(usually loaded at boot time, e.g., via DOS's CONFIG.SYS or MS-Windows

registry/WIN.INI/SYSTEM.INI), or with removable host drivers (e.g., a

DOS TSR like USBUHCI).  It is desirable for the user to have complete

control of if and when a particular USB host controller will be enabled.

This API includes functions that allow communication between different

host drivers, and between host drivers and device/interface drivers, to

allow individual host drivers to be added/removed/disabled/enabled at

any time by the user.

BIOS-LEVEL USB API 0.05            Page 4 of 168

This particular version of the API is written with intimate knowledge of

only one particular type of USB host controller, specifically UHCI

(Universal Host Controller Interface).  There are two other major types

of host controllers currently in mass production: OHCI (Open Host

Controller Interface) and EHCI (Enhanced Host Controller Interface, for

USB 2.0 hosts).  There are also two types of Host Controllers under

development, but not yet in mass production: WHCI (Wireless Host

Controller Interface, for USB 2.5 (USB Certified Wireless) hosts), and

XHCI (Extensible Host Controller Interface, for USB 3.0 hosts).  As

actual programs are developed to control those particular types of

hosts, this API will be modified accordingly.  It is expected that the

changes needed in this API to be able to accommodate OHCI hosts will be

relatively minor.  Changes required to accommodate other types of hosts,

however, are expected to be significant.

This document is written assuming the reader has a basic knowledge of

USB architecture, which, to be perfectly frank, is pretty complicated.

If you don't understand the relationship between USB terms like host,

frame, device, interface, alternate interface, end point, configuration,

class, subclass, protocol, vendor, product, descriptor, isochronous,

bulk, control, interrupt, report, in, out, etc., you'll need to read a

USB specification or other similar document.  At times in this document,

certain of these items will be described superfluously, but definitely

not in enough detail for you to understand USB just by reading this API

specification.  In fact, reading this document without some pre-existing

knowledge of the USB architecture may frustrate and confuse your

understanding of USB more than it helps you.

BIOS-LEVEL USB API 0.05            Page 5 of 168

GENERAL NOTES ABOUT USB

�����������������������

The USB architecture is a master/slave (polled), tree-shaped, packet-

based architecture.  Packets of data are scheduled to get transferred

across the bus, to be sent/received by an end point on a

device/interface.  The scheduling request is issued to the host driver,

which will attempt to transfer the packet across the bus hardware at the

next available polling interval.  The host driver is in complete control

of when and how everything happens on the bus.  Devices attached to the

bus, and the device/interface drivers that control those devices, can

only do things with the bus when the host driver lets them.

One concept in USB that can be quite confusing is the difference between

a device and an interface.  What most people would commonly think of as

a "device" is usually (but not always) called an "interface" in USB.

For example, a keyboard and a mouse that are separate from each other

are considered two separate devices, and each one has a single

interface.  A combination keyboard/mouse is considered a single device,

but has two different interfaces, which must be controlled separately.

A USB modem has two interfaces, one for the actual transfer of data and

one for control of the modem (usually, control is performed with AT

commands similar to a regular modem).  With a regular modem, though, you

"toggle" back and forth between the data and control modes (usually with

a "+++" command).  In the USB architecture, the "things" you control are

usually called interfaces, not devices.

Another thing that be quite confusing is the distinction (or lack

thereof) between a piece of hardware (a host controller or a bus-

attached device, like a remote hub or a mouse), and the software (host

driver or device/interface driver) that enables a device to actually be

used for something.  In this document, we will mostly be addressing the

software (driver) that "enables" a particular hardware device.  However,

because it takes both hardware and software to be able to do something

useful, in this document we may at times use terms like "host

controller" (the hardware) and "host driver" (the software)

interchangeably.  We will generally try to use the appropriate terms at

the appropriate times, so hopefully things will not get too confusing.

The USB architecture, and USB devices in general, is notoriously

unreliable.  There are some things that simply don't work correctly the

first time you try them, and you need to just keep trying over and over

again until it finally works.  Host drivers will, by default,

automatically repeat (several times) processes that are known to be

unreliable.  For example, all devices must go though a complicated

initialization and enumeration process after they are first attached to

the bus.  The initialization and enumeration process is unreliable, and

the host driver will automatically repeat the entire process for a

device up to 5 times before finally giving up.  This is discussed in

detail below for the various situations where it applies.

BIOS-LEVEL USB API 0.05            Page 6 of 168

OWNERSHIP REGISTRATION

����������������������

At the core of this API is a concept called "ownership registration."

Understanding ownership registration is absolutely fundamental to

understanding how the API works -- if you don't get a grasp of ownership

registration, the API will make no sense at all.

A host driver controls and monitors a particular USB bus and everything

attached to that bus.  However, there are certain aspects (entities) of

a bus that can be "owned" by a piece of software external to the host

driver.  There are three different entities that can be owned by

external software under this API: devices newly attached to the bus

(also called device 0's), interfaces, and frame timing.

The purpose of registering as the owner of an entity under the API is

two-fold.  First of all, it provides a mechanism for the host driver to

notify the entity owner when something has happened to the entity

itself, or when something has happened on the bus that indirectly

affects the entity.  For example, if a piece of software has registered

as the owner of a particular interface on a device, the interface owner

is notified by the host driver when the user disconnects the device from

the bus.  The interface owner is also notified by the host driver if the

user disconnects an upstream device (hub) that (indirectly) results in

the owned interface also being disconnected from the bus.  The

NOTIFICATION CODES table below (page 10) lists all of the different

reasons a registered owner might be notified.

The second reason for ownership registration is that it provides a form

of "protection".  That is, only the owner of any particular entity is

allowed to do things to control that entity.  For instance, only the

registered owner of the frame timing is allowed to do anything that

changes the frame timing.  Other software is allowed to monitor what the

registered frame timing owner does to the frame timing, but only the

owner is allowed to actually make changes.

Having more than one piece of software own (control) an entity on the

bus would almost guarantee disaster, so this API does not allow that to

happen.  However, because of limitations in the way the bus is designed,

it is possible for pieces of software (or the user) to do things that

directly or indirectly affect entities that they do not own.  The

ownership registration process minimizes those kinds of "protection

violations", but, unfortunately, cannot eliminate them completely.

BIOS-LEVEL USB API 0.05            Page 7 of 168

An entity owner registers by issuing an appropriate INT 14h request,

which must include a call back address and a user packet ID.  The INT

14h request will return with the carry flag indicating success or

failure (CF = clear if success), an error code in AX (AX = 0 if

success), and request-specific data in BX, CX, and DX.  Typically, part

of the request-specific data returned will be a handle number in the CX

register.  This handle number will be required for later requests, such

as when it becomes necessary to un-register as the entity owner.

After the owner of an entity has registered, the owner will be notified

by the host driver(s) whenever anything happens that might affect how it

operates (device connections and disconnections, frame timing changes,

the host starting and stopping, etc.).  The complete list of

NOTIFICATION CODES is detailed below on page 10.  The owner is notified

by a call to the call back address that was provided during the original

INT 14h ownership registration request, with the appropriate

notification code in AX, the user packet ID (the same one that was

provided by the entity owner during the original ownership registration

request) in BX, and notification-specific data in CX and DX.  The rest

of the "environment" related to the notification call (CPU flags, stack,

etc.) is as detailed below in the "CALL BACK ADDRESSES & USER PACKET

ID's" section (page 15).

As noted before, this API is designed to operate almost completely as a

background process.  The way it does this is by "tying" itself to

hardware interrupts (IRQ's).  When a notification occurs, it is almost

always being called from "inside" an IRQ processing routine.  This is of

major concern if you are attempting to write a piece of software to

interface with this API.  IRQ processing routines are very time-

sensitive, and any program that is "tied" to a hardware IRQ needs to

concern itself with efficiency.  If you are not accustomed to writing

software for hardware IRQ processing, you may be in for some surprises.

BIOS-LEVEL USB API 0.05            Page 8 of 168

Let us briefly discuss each of the three types of ownership registration

entities (new devices, interfaces, and frame timing).  When a piece of

software registers as a new device (device 0) owner, it is telling the

host driver(s) that it wants to be notified immediately when a certain

kind of device/interface is first attached to the bus.  As part of the

"Register As Device 0 (New Interface) Owner" request (detailed below on

page 123), the owner provides a profile (vendor ID, product ID, device

class/subclass/protocol, and interface class/subclass/protocol) of the

device(s)/interface(s) it would like to be notified of.  For example, an

owner could choose to be notified of every new device/interface that

gets attached to the bus, or only certain devices/interfaces (e.g., all

human interface devices, all mice, all Logitech mice, or just Logitech

model XXX mice).  If a new device is attached to the bus, but none of

its interfaces match the device 0 owner's profile, the owner will not be

notified.  Note that a device/interface driver is still able to "find"

all of the appropriate interfaces it may want to own, even if it is not

registered as a device 0 owner.

The second type of ownership registration is registering as the owner of

an interface.  When a piece of software registers as an interface owner,

it is telling a host driver that it "owns" one particular interface on

one particular device.  The owner will be notified of anything that

happens on the bus that might affect the operation of the interface

(device disconnects, resets, enables/disables, etc.).  Although it is

not enforced by the host driver, the owner of an interface is the only

software allowed to modify anything functional regarding an interface

(e.g., selecting an alternate interface).  In addition, the owner is the

only software allowed to download/upload any "real" data from/to the

interface (typically, "real" data comes from an endpoint other than

endpoint 0).  The "Register As Interface Owner" request is explained in

detail below on page 121.

The last type of ownership registration is registering as the frame

timing owner.  When a piece of software registers as the frame timing

owner, it is the only software allowed to speed up or slow down the bus.

The bus nominally runs at 12 Mbps, but can be adjusted to run slightly

slower or slightly faster than that.  The only time it should be

necessary to adjust the frame timing is if there is an isochronous

device attached to the bus, and that isochronous device must be

synchronized to an external clock of some sort.  Details on the

"Register As Frame Timing Owner" request are given below on page 57.

BIOS-LEVEL USB API 0.05            Page 9 of 168

NOTIFICATION CODES

(Sent to Entity Owners regarding Bus/Device Changes)

���������������������������������������������������������������������ͻ

�       Mnemonic       �Value�             Description                �

���������������������������������������������������������������������͹

�OwnerCallNewDvc       �0001h�New Device Attached                     �

�OwnerCallDvcDisc      �0002h�Registered Device Disconnected          �

�OwnerCallDvc0Disc     �0003h�Unregistered Device Disconnected        �

�OwnerCallDontLook     �0007h�Stop Looking for New Interfaces         �

�OwnerCallDvcEnable    �0008h�Device Enabled                          �

�OwnerCallDvcDisable   �0009h�Device Disabled                         �

�OwnerCallDvcReset     �000Ah�Device Resetting                        �

�OwnerCallDvcSuspend   �000Bh�Device Suspended                        �

�OwnerCallDvcResume    �000Ch�Device Resumed                          �

�OwnerCallDvcPwrOn     �000Dh�Device Powered On                       �

�OwnerCallDvcPwrOff    �000Eh�Device Powered Off                      �

�OwnerCallDvcPwrReset  �000Fh�Device Power Reset                      �

���������������������������������������������������������������������Ķ

�OwnerCallNewHost      �0011h�New Host Driver Installed               �

�OwnerCallHostDisc     �0012h�Existing Host Driver Uninstalled        �

�OwnerCallHostHWErr    �0017h�Host Hardware Error (Host will be Reset)�

�OwnerCallHostRun      �0018h�Host Running                            �

�OwnerCallHostStop     �0019h�Host Stopped                            �

�OwnerCallHostReset    �001Ah�Host Resetting                          �

�OwnerCallHostSuspend  �001Bh�Host Suspended                          �

�OwnerCallHostResume   �001Ch�Host Resumed                            �

�OwnerCallHostDebug    �001Dh�Host in Debug Mode                      �

�OwnerCallHostSST      �001Eh�Host Single-Stepped                     �

�OwnerCallTimingChg    �0021h�Host Frame Timing Changed               �

���������������������������������������������������������������������Ķ

�HubCallEnumerated     �0077h�New/Reset Device has been Enumerated    �

�HubCallEnable         �0078h�Enable Device                           �

�HubCallDisable        �0079h�Disable Device                          �

�HubCallReset          �007Ah�Reset Device                            �

�HubCallSuspend        �007Bh�Suspend Device (Selective Suspend)      �

�HubCallResume         �007Ch�Resume Device                           �

�HubCallPwrOn          �007Dh�Power On Device                         �

�HubCallPwrOff         �007Eh�Power Off Device                        �

�HubCallPwrReset       �007Fh�Power Reset Device                      �

���������������������������������������������������������������������Ķ

�OwnerCallDvcDiscUS    �0082h�Device Disconnected Upstream            �

�OwnerCallDvcEnableUS  �0088h�Device Enabled Upstream                 �

�OwnerCallDvcDisableUS �0089h�Device Disabled Upstream                �

�OwnerCallDvcResetUS   �008Ah�Device Reset Upstream                   �

�OwnerCallDvcSuspendUS �008Bh�Device Suspended Upstream               �

�OwnerCallDvcResumeUS  �008Ch�Device Resumed Upstream                 �

�OwnerCallDvcPwrOnUS   �008Dh�Device Powered On Upstream              �

�OwnerCallDvcPwrOffUS  �008Eh�Device Powered Off Upstream             �

�OwnerCallDvcPwrResetUS�008Fh�Device Power Reset Upstream             �

���������������������������������������������������������������������ͼ

BIOS-LEVEL USB API 0.05            Page 10 of 168

IMMEDIATE, SCHEDULE, AND LARGE TRANSACTIONS

�������������������������������������������

There are many different types of INT 14h requests in this API.  Some

INT 14h requests initiate transactions that are relatively simple and

straightforward, while others initiate transactions that are extremely

complicated and time-consuming.  It is convenient to classify INT 14h

requests into three different "categories": immediate, schedule, and

large.  These categories have to do with how and when the transaction

initiated by the INT 14h request is truly "completed."

Immediate transactions are like most INT transactions you would deal

with on a regular basis: they are completely finished as soon as the INT

14h request returns.  The INT 14h request will return with the carry

flag indicating success or failure (CF = clear if success), an error

code in AX (the complete ERROR CODE table is detailed below on page 35),

and some sort of transaction-specific data in BX, CX, and DX.

Just like immediate transactions, schedule transactions and large

transactions also return from the INT 14h request with the carry flag

indicating success or failure, AX containing an error code, and

transaction-specific data in BX, CX, and DX.  However, a successful

schedule or large transaction is not actually complete when the INT 14h

request returns.  When successful schedule or large transactions return

from the INT 14h request, a background process has been scheduled

(usually on the USB bus).  The background process will actually be

completed at some other (usually much later) point in time.  Exactly

when the background process might complete depends on several factors,

including the specific type of transaction and the current state of the

USB hardware and bus.

As part of the INT 14h request to initialize the schedule or large

transaction, the requester provides a call back address (segment:offset)

that will be called (notified) when the background process has been

completed.  In addition to a call back address, the requester also

provides a user packet ID.  A user packet ID is simply a word (two

bytes) that is "returned" to the requester at the end of the process (in

the BX register) when the call back address is called.  The user packet

ID can be an index, a pointer, or whatever the requester may want it to

be.

BIOS-LEVEL USB API 0.05            Page 11 of 168

The difference between a schedule transaction and a large transaction

lies in exactly how complicated the scheduled background process is.

With a schedule transaction, what is scheduled is a single packet

transaction on the bus (in USB terminology, a single transfer descriptor

is created).  A transfer descriptor (TD) will complete with one or more

status codes (the complete STATUS CODE table is detailed below on page

14).  Since a TD can actually complete with more than one status code at

the same time, the individual status codes are bit-mapped and are OR'd

together to get the complete TD status code for a schedule transaction.

At the end of a schedule transaction, the call back address is called

with the complete TD status code in AX, the user packet ID in BX, and

transaction-specific data in CX and DX.

A large transaction is very similar to a schedule transaction.  However,

what a large transaction schedules to occur in the background is not

simply a single USB packet (transfer descriptor or TD).  Instead, what

is scheduled by a large transaction is a complicated series of "sub-

transactions."  This may include one or more schedule-type transactions

(transfer descriptors), it may include other types of "sub-processes"

that do not necessarily even involve packets traversing the bus, and may

also include some "logic" so that exactly what happens depends on the

current state of something related to the bus.  What happens during a

large transaction is, in fact, so complicated that it is not possible to

generate a status code at the end of it that has any specificity.  That

is, the status code at the end of a large transaction will either be 0

(TDStsOK) or 8000h (TDStsLargeCallErr), with no further details

available as to why the transaction might have failed.  Just as in a

schedule transaction, at the end of a large transaction, the call back

address is called with the status code in AX, the user packet ID in BX,

and transaction-specific data in CX and DX.  Other than the specificity

of the status code during the call back, a large transaction and a

schedule transaction appear pretty much the same to the requester.

There are certain states of the bus where certain kinds of schedule and

large transactions can actually be completed immediately, and do not

actually need to be "scheduled".  An example of this would be a request

to start the host running (see page 48 below), when the host is already

running.  In such a situation, it is possible (in fact, likely) that the

transaction call back process will actually occur BEFORE the INT 14h

request is completed.  This is backwards from what normally happens

(typically, the call back process occurs at some point in time much

later than the INT 14h request returns).  If you are writing a program

to interact with this USB API, this can be a source of extremely-hard-

to-find (and typically very intermittent) program bugs.  Consider

yourself forewarned.

BIOS-LEVEL USB API 0.05            Page 12 of 168

There is also one particular status code that is "special", and works a

little bit differently than the rest: TDStsDvcRemoved (2000h).  This

particular status code is returned when the host driver detects that a

device has been removed from the bus (physically unplugged or reset)

while the transfer descriptor (TD) was in progress, thereby making it

impossible for the TD to complete.  There are three things that are

"special" about TDStsDvcRemoved.  Firstly, the TDStsDvcRemoved status

code is not combined (OR'd) with any other status codes at the same

time.

Secondly, when a TD returns with a status code of TDStsDvcRemoved in AX,

CX and DX are always returned as 0.  For most transactions, CX contains

the number of data bytes that were successfully transferred by the TD.

When a device is disconnected while a TD is in progress, some or all of

the data associated with the TD may, in fact, have been transferred

successfully before the disconnect occurred.  Unfortunately, the host

driver has no way of knowing how many bytes were actually transferred

unless it can communicate with the device, which is impossible since the

device is no longer attached to the bus.  Therefore, the host driver

must assume that no data was able to be transferred, and so returns with

CX = 0.

Lastly, when a TD returns with a status code of TDStsDvcRemoved, the TD

is automatically unscheduled and deleted, even if it is a periodic

transaction (periodic interrupt or isochronous transaction).  Normally,

periodic transactions can only be terminated with a "Close Transaction

Handle" request (page 147)., but the host driver performs this function

automatically when a device is disconnected from the bus.

BIOS-LEVEL USB API 0.05            Page 13 of 168

STATUS CODES

(Sent to Transfer Descriptor (TD) Requester at end of Transaction)

(Multiple Errors are OR'd together)

�������������������������������������������������������������������ͻ

�     Mnemonic      �Value�               Description               �

�������������������������������������������������������������������͹

�TDStsOK            �0000h�ACK Received (Complete/No Errors)        �

�������������������������������������������������������������������Ķ

�TDStsNAKReceived   �0001h�NAK Received                             �

�TDStsStalled       �0002h�Stalled                                  �

�TDStsTimeout       �0004h�Timed Out (Bulk/Control)                 �

�TDStsOverDue       �0008h�Overdue (Isoch/Interrupt)                �

�������������������������������������������������������������������Ķ

�TDStsShortPacket   �0010h�Short Packet Detected                    �

�TDStsBabbleDetected�0020h�Babble Detected                          �

�TDStsCRCTOReceived �0040h�CRC/Timeout Error Received               �

�TDStsBitStuffError �0080h�Rx Data contained > 6 ones in a row      �

�������������������������������������������������������������������Ķ

�TDStsDataBuffErr   �0100h�Data Buffer Error (Overflow/Underflow)   �

�������������������������������������������������������������������Ķ

�TDStsControlSetup  �1000h�Error occurred during Control Setup Stage�

�TDStsDvcRemoved    �2000h�Device Removed while TD was in progress  �

�TDStsLargeCallErr  �8000h�Error During Large Call                  �

�������������������������������������������������������������������ͼ

The codes in this status code table can come from any of several

different sources.  Many of the status codes are caused by hardware- or

bus-related problems.  For those kinds of errors, the host driver is

simply "passing through" the same error information that it received

from the host hardware.  The codes that fall into this category are

TDStsNAKReceived, TDStsStalled, TDStsShortPacket, TDStsBabbleDetected,

TDStsCTCTOReceived, TDStsBitStuffError, TDStsDataBuffErr, and

TDStsControlSetup.

TDStsDvcRemoved is generated when a device is removed (unplugged or

reset) from the bus.

The remaining status codes (TDStsTimeout, TDStsOverDue, and

TDStsLargeCallErr) are generated by the host driver itself.  These

status codes are not necessarily directly related to problems on the bus

hardware (though they can be indirectly related) -- many times these

status codes are caused by unresponsive devices attached to the bus.

BIOS-LEVEL USB API 0.05            Page 14 of 168

CALL BACK ADDRESSES & USER PACKET ID's

��������������������������������������

One fairly common theme in this API are the concepts of call back

addresses and user packet ID's.  These things are required for certain

types of INT 14h transactions: Ownership registrations (device 0,

interface, and frame timing), as well as schedule transactions and large

transactions.  While these types of transactions may appear to be very

different from each other, the CallBack/Notification process is handled

in pretty much the same way from a technical perspective.  That process

is discussed here.

We do, however, need to separate the call backs into two different

"categories": transaction status call backs (schedule transactions and

large transactions), and ownership notification call backs (device 0,

interface, and frame timing ownership registration notifications).  This

is necessary for a couple of different reasons.  First of all, it is

possible to provide a "null" or "fake" call back address (exactly how to

do this is discussed a couple of paragraphs down - keep reading).  A

null call back address is "ignored" at the time of the

callback/notification - that is, the call back process never actually

occurs.  Most of the time, the requester of a transaction will want to

know when a background process is complete, but (believe it or not)

there are times when it is actually quite useful to not know.

You can provide a null address for a transaction status call back, but

the address for an ownership notification call back can NOT be null (it

must be a real address that actually does something).  Ownership

notifications are discussed above in the "OWNERSHIP REGISTRATION"

section (page 7), and are issued when various kinds of events happen on

the bus that may affect how a device or program operates.  The owner

notification call back address is called, for instance, when an

appropriate device is connected or disconnected from the bus, if the

host is stopped for some reason, if the bus is put into suspend mode,

etc.  A legitimate ownership notification call back address must be

provided.

The second reason we need to differentiate between ownership

notification call backs and transaction status call backs are because of

the types of codes that get issued during the call.  During a

callback/notification, the user packet ID is in the BX register, call-

specific data is in the CX and DX registers, and there is a code in the

AX register.  For ownership notifications, the code in the AX register

is a notification code (see the NOTIFICATION CODE table above on page

10).  For a transaction call back, the code in the AX register is a

status code (see the STATUS CODE table above on page 14).

BIOS-LEVEL USB API 0.05            Page 15 of 168

A call back address must always be provided in a standard segment:offset

format.  A null call back address (an "ignored" call back address) can

either have a segment of 0000h, or an offset of FFFFh (neither of which

would exist in a real, functional code address).  If the segment is

anything other than 0000h, and the offset is anything other than FFFFh,

the call back address is considered legitimate.

Immediately before calling the callBack/notification code, the host

driver sets up the following "environment" for the callBack/notification

code to run in.  The host driver performs the following procedures

(though not necessarily in this order) immediately before calling the

callBack/notification address:

1. If the call back address is null (segment = 0000h or offset =

FFFFh), does nothing.

2. Saves CPU flags register (PUSHF).

3. Saves all general CPU registers ({E}AX, {E}BX, {E}CX, {E}DX,

{E}DI, {E}SI, {E}BP, and {E}SP).

4. Saves all segment registers (DS, ES, {FS}, {GS}, SS).

5. Sets AX, BX, CX, and DX registers as appropriate for the call: AX

contains the status or notification code, BX contains the user

packet ID, and CX and DX contain transaction-specific or

notification-specific values.

6. Sets string functions to go forward (CLD).

7. Enables interrupts (STI).

8. Initializes zero flag and sign flag (OR AX,AX).

9. Sets all segment registers, except SS, to the same value: The

segment (CS) of the call back address (DS = ES = {FS} = {GS} =

CS).

10. All that is expected of the call back code is to preserve stack

integrity and end with a RETF.  The call back code does not need

to preserve registers or flags.

11. The call back code will almost always be called as part of a

hardware interrupt (IRQ) processing routine, which makes it speed-

sensitive.  IRQ processes should be completed as quickly as

possible in order to keep the computer running efficiently.  If

the call back code can consume a significant amount of time, it is

usually best to simply have the call back code set some kind of

semaphore flag or counter indicating that it has happened.  The

time-consuming part of the code can be performed at some later

point in time, outside the critical timing constraints of the IRQ.

BIOS-LEVEL USB API 0.05            Page 16 of 168

INTERACTION BETWEEN MULTIPLE HOST DRIVERS

�����������������������������������������

It is possible for there to be several host controllers installed on the

same computer, and each one will need a complementary host driver to

enable it.  In theory, it would be possible to have a single, large,

monolithic host driver capable of enabling all different types and

quantities of host controllers, but this would not be a practical

solution.  Instead, it is much more efficient to install several

independent host drivers, one for each host controller.  From the

perspective of the outside world, though, the API must provide a single,

uniform interface that makes it appear as though there is only one

monolithic driver.

The API services are provided as an extension to the BIOS serial

services interrupt, INT 14h.  Like all interrupts, INT 14h is chained,

and whenever a new host driver is installed into memory it becomes the

first program to receive an INT 14h request when it is issued.  In

addition, it is possible for the user to uninstall or disable any of the

host drivers at any time, even if they are not the most recent host

driver to be installed.  Therefore, each individual host driver must be

capable of being the only host driver installed, as well as being the

first, middle, or last host driver in a "chain" of installed host

drivers.  In order to accomplish this, every host driver must have some

basic knowledge of every other host driver that is installed.

The first host driver to receive a USB-related INT 14h request (usually

the last host driver that was installed into memory) is responsible for

certain error-checking functions.  A USB-related INT 14h request has

several data fields containing details about the exact type of request,

one of which is the host index.  The API allows for a total of 16 host

drivers, with specific host indexes numbered from 0-15.  The first host

driver to receive a request for a specific host index is responsible for

checking the validity of the requested host index, even if the first

host driver is not the host driver associated with the host index.  If

the host index is invalid, the first host driver to receive the request

is responsible for returning the appropriate error code to the calling

program.  If the host index is valid, and the first host driver is the

driver associated with the index, it will then complete the processing

of the request.  If the host index is valid, but the first driver is not

the one associated with the index, it will pass the request down the INT

14h chain to the appropriate driver for processing.  An invalid request

must not be allowed to "pass through" the INT 14h chain to a non-USB

interrupt handler, where it will be improperly processed.

BIOS-LEVEL USB API 0.05            Page 17 of 168

In addition, there are certain types of USB-related INT 14h requests

that are not directed at a specific host index, but rather are directed

at any or all host drivers.  The host index for such requests is 255,

rather than a number between 0 and 15 as it is for specific hosts.

There are certain types of requests that are directed at ANY host

driver, and must be processed by the first host driver with no "help"

from other host drivers.  An example of this is the "Get Host Hardware

Information" request with a host index of 255, which is discussed below

on page 44.

There are also certain types of requests that are directed at ALL host

drivers (rather than ANY host driver), and can be thought of as a

"broadcast request".  When one of these requests is issued, the first

host driver to receive the request is responsible for "translating" the

request into an appropriate sub-request, and then issuing individual

sub-requests to each separate host driver through separate INT 14h sub-

Requests.  An example of this is the "Register As Device 0 (New

Interface) Owner" request, and the related "InterHost Register As Device

0 Owner" sub-request (discussed below on pages 123 and 159,

respectively).

A very important and integral part of this API is the process of

ownership registration, wherein a device/interface driver registers as

the owner of an entity (this is discussed in great detail in the

"OWNERSHIP REGISTRATION" section above on page 7).  A piece of software,

external to the host driver(s), can "own" certain new devices that get

attached to the bus (device 0 ownership).  A piece of software that

"owns" a newly attached device (device 0) is automatically informed by

the host driver(s) when an appropriate new device is attached to the

bus.

There is a special "new device ownership table" that keeps track of all

of the device 0 ownership registrations, and it has a specific format

(detailed in the "Copy The New Device Ownership Table" request below,

page 163).  Remember that individual host drivers can be added/removed

from memory at any time, so each host driver must be capable of being

the only, first, middle, or last host driver in memory at any time.

Therefore, every host driver must maintain its own copy of the new

device ownership table, and every copy in every host driver must be

EXACTLY the same as all of the others.  Several requests in the API are

specifically designed to allow different host drivers to "share" the

contents of the new device ownership table.  They are detailed below in

the "TRANSACTION REQUEST TYPES" section (starting on page 42).

BIOS-LEVEL USB API 0.05            Page 18 of 168

If a host driver is the only host driver remaining in memory, and there

are any valid entries in the new device ownership table, the host driver

cannot be removed from memory.  Doing so could cause the computer to

crash, since a device 0 owner could issue a faulty request under the

assumption that at least one host driver (and consequently this API) was

still installed in memory.  Although device 0 owners are notified when

host drivers are added and removed from memory, they are not required to

keep track of the number of host drivers currently installed.  In

particular, they are not required to "know" that the last host driver

has been removed, and that consequently the API is no longer installed.

The last host driver cannot be removed from memory if there is any

possibility that another "dependent" program might be under the

assumption that the API is still installed.

As an example of the interaction between multiple host drivers, here are

the general steps that a new host driver must go through as it is being

initialized (assuming that it is not the first host driver in memory):

1.  The new host driver issues an "Install Check" request (page 33), to

determine whether or not the API is already installed (whether or

not there is at least one other host driver already installed in

memory).

2.  The new host driver issues a "Get Host Hardware Information" request

with I14RHostIndex = 255 (page 44), to determine which host indexes

are already being used by other host drivers.  The new host driver

"grabs" an unused host index for itself to use.

3.  The new host driver issues a "Copy the New Device Ownership Table"

request (page 163), to obtain a copy of the new device ownership

table from the existing host driver(s).

4.  The new host driver assumes control of INT 14h, and becomes the

first USB host driver to receive INT 14h requests.

5.  The new host driver issues a "New Host Driver Has Been Installed"

request (page 157), to inform all the other host drivers, and

consequently all device/interface drivers, of its existence.

BIOS-LEVEL USB API 0.05            Page 19 of 168

IRQ's

�����

Each USB host controller requires an IRQ (hardware Interrupt ReQuest) to

operate.  There are a very limited number of IRQ's available on a

computer (16).  Several of the IRQ's are dedicated to specific

functions, and cannot be used by USB host controllers.  Most modern

computers have several host controllers (my newest computer has eight of

them), and it is rarely possible for each USB host controller to have

its own dedicated IRQ (there simply aren't enough IRQ's to go around).

Rather, two or more USB host controllers must many times "share" the

same IRQ.  In addition, at least on some computers, an IRQ used for a

USB host controller must also be shared with some other (non-USB)

hardware, such as a FireWire or BlueTooth controller.

"Sharing" IRQ's is a tricky proposition in DOS.  There has been a long-

standing rule-of-thumb with DOS programmers that states that IRQ's

cannot be shared, though that is not at all true.  IRQ's can be shared

in DOS, as long as the software is written correctly.  USB host driver

software MUST be designed to share IRQ's.

BIOS-LEVEL USB API 0.05            Page 20 of 168

HUBS

����

A USB hub is a very interesting type of device.  It is in one respect

simply another device that can be attached to the bus, but is also very

special because it is the mechanism that allows the physical and logical

extension of the bus to other devices.  Host drivers "extend" their

control of the bus, and all devices attached to the bus, by being able

to control the hubs.

To enable, disable, reset, suspend, resume, power on, or power off a

particular device, the host driver does not actually do anything

directly to the device it is trying to control.  Instead, what the host

driver does is do something to the port of the upstream hub that the

device is attached to.  For example, let's assume that the user wants to

disable a particular device.  Here are the steps that actually occur in

order to make that happen:

1.  A "Disable Device Given Device Address" request (page 102) is issued

to the host driver.  Among other things, the request contains the

address of the device to be disabled, and a call back address to be

notified when the disable process (which runs in the background) is

complete.

2.  The host driver internally "translates" the device address from the

request to a hub address and port number (the port that the device

is attached to).  The host driver notifies the appropriate hub

interface owner (the hub driver), with notification code =

HubCallDisable.  The notification includes the host index, hub

address, hub port number, and a large call index.

3.  The hub driver issues a "Schedule Control (Setup) Transaction"

request (page 139) to the host driver, sending a control packet

across the bus to the hub hardware telling it to disable the port.

4.  The hub driver issues another "Schedule Control (Setup) Transaction"

request (page 139) to the host driver, sending a control packet

across the bus to the hub hardware to verify that the port has

actually been disabled.

5.  The hub driver issues a "Large Call Completed" request (page 154) to

the host driver.  The request contains a status code indicating

success or failure of the hub driver's attempt to disable the port.

The request also "returns" the large call index back to the host

driver.

6.  The host driver informs (calls back) the original INT 14h requester

of the status of the request (status code = TDStsOK or

TDStsLargeCallErr).

BIOS-LEVEL USB API 0.05            Page 21 of 168

7.  If the device was successfully disabled, the host driver notifies

all interface owners on the device that the device has been disabled

(notification code = OwnerCallDvcDisable).

8.  If the device was successfully disabled, and the device is a hub,

the host driver notifies all interface owners of all devices

downstream of the hub that an upstream device has been disabled

(notification code = OwnerCallDvcDisableUS).

This example hopefully illuminates the complicated interactions that

take place between host drivers and hub drivers.  There are special

kinds of messages that are sent back and forth between host drivers and

hub drivers, which are not needed for any other type of device.  The

NOTIFICATION CODE table above (page 10) lists all of the different codes

that a host driver may use to notify an interface owner.  There is a

special section of the table, where the mnemonic starts with "HubCall"

(instead of "OwnerCall"), that only applies to hub interface owners.

The HubCall notifications are special notifications that are needed to

be sent by a host driver to a hub driver in order to control the bus.

There is also a special INT 14h request that is issued by a hub driver

to a host driver, that is not needed for any other type of device except

a hub.  It is the "Send Hub Characteristics to Host Driver" request, and

is detailed below on page 71.  Immediately after a hub driver registers

as owner of the hub (immediately after it has issued the "Register As

Interface Owner" request), it must issue a "Send Hub Characteristics to

Host Driver" request to notify the host driver of certain power-related

characteristics of the hub.  There is a more detailed discussion of this

in the "Power Distribution" section below (page 26).

BIOS-LEVEL USB API 0.05            Page 22 of 168

DEVICE ENUMERATION

������������������

There is a complicated process in the USB architecture called device

enumeration.  This is the process where a device newly attached to a bus

is recognized, initialized, configured, and assigned an address.  Every

device attached to the bus must go through the enumeration process

before it can be used.

Under this API, there are three different ways to initiate the

enumeration process.  The first way is to physically attach (or

disconnect and then re-attach) a device to an open port on a hub.  This

is the "standard" way to start the enumeration process.

The second way to initiate the enumeration process is to issue a "Reset

Device Given Device Address" request (page 104).  Resetting a device

that is already attached to the bus is functionally equivalent to

physically disconnecting and then re-attaching a device, but does not

require physical manipulation of the device (which can sometimes be a

HUGE pain in the backside).

The last way to begin the enumeration process is to issue a "Reset

Device and Set New Configuration" request (page 114).  This is

functionally similar to a simple reset (discussed in the previous

paragraph);  however, a Set New Configuration request allows the user to

select the configuration value that the device will have after the

enumeration process is complete, while a simple reset will only set the

default configuration value (one) for the device.

Regardless of how the enumeration process is initiated, the process

itself is rather complicated.  It involves a high degree of interaction

between the host driver and hub driver of the hub that the new device is

attached to.  Following is a summary of that process:

1.  The hub driver is continually polling the hub hardware for changes

in status.  When the hub hardware detects that a new device has been

attached to one of its ports, it informs the hub driver at the next

opportunity (during the next status poll).  If a device was recently

reset, it will look like a newly attached device to the hub

hardware.

2.  The hub driver verifies that the new device is disabled by the hub

hardware, and waits at least 100 milliseconds for power to stabilize

on the new device.  The hub driver then verifies that the new device

is still attached to the hub hardware, and obtains the speed (low,

full, or high) of the new device from the hub hardware.

3.  The hub driver then issues a "Notify Host Driver of New Device

Connection" request (page 67) to the host driver.

4.  The host driver initializes its "enumeration retry counter" to five.

BIOS-LEVEL USB API 0.05            Page 23 of 168

5.  The host driver notifies the hub driver to reset the new device,

which in turn causes the hub driver to issue a control request back

to the host driver, which sends a control packet across the bus to

the hub hardware to perform the reset.

6.  The host driver issues several control requests to the new device to

assign it an address, and to obtain basic information about the

device (e.g., the host driver downloads the device, configuration,

interface, and end point descriptors from the new device).

7.  The host driver issues a control request to the new device to set

its configuration value.  The default configuration value for a new

device is one, but other configuration values can be assigned using

the "Reset Device and Set New Configuration" request (page 114).

8.  If the new device supports the remote wakeup feature, the host

driver issues a control request to the new device to enable the

feature.

9.  If the entire enumeration process was successful, the host driver

issues a notification to the hub driver to inform the hub driver

that the enumeration process is complete.  Part of the notification

to the hub driver includes the address of the new device (a number

between 2 and 127).  If the device is configured (has a

configuration value other than zero), the host driver notifies all

device 0 owners (registered in the new device ownership table) whose

profiles match the attributes of the new device, informing the

device 0 owners of the new device attachment.

If the enumeration process did not work correctly for any reason,

the host driver decrements the "enumeration retry counter".  If the

counter has not reached zero, the host driver waits three seconds

and then repeats this entire process beginning at stage 5.

If the counter has reached zero 0 (the enumeration process has been

tried five times with no success), the new device is declared to be

"bad" by the host driver.  The host driver notifies the hub driver

to disable the device, which in turn causes the hub driver to issue

a control request to the hub hardware (via the host driver) to

disable the port that the new device is attached to.  The host

driver also notifies the hub driver that the device is bad, by

telling the hub driver that the device has address 255 instead of

telling it that it has a legitimate address (between 2 and 127).

BIOS-LEVEL USB API 0.05            Page 24 of 168

A host driver must "assign" an address to a bad device, even though the

host driver is the only entity that knows what the "fake" address is.

The hub driver that is controlling the hub that the device is attached

to will never know what the address is, since the hub driver is told

that the "bad" device has address 255 (which is an "illegal" address).

Having a fake address assigned by the host driver allows the user to

still manipulate the bad device, performing actions such as powering off

or resetting the device.  The host driver must respond to all requests

regarding the bad device, returning information much as if the device

were OK.  Of course, the host driver may not know much, if anything,

about the bad device, so the host driver must in some cases return

"fake" information about the device (returning all zeroes in the status

registers it does not actually know).  For example, if a "Get Device

Class Information" request (described below on page 96) is issued for a

bad device, the host driver will return successfully, but values in

status registers (including the USB version returned in the CH register)

will be 0 if the host driver was unable successfully download the device

descriptor from the device.  The "Get Device Status Information" request

(discussed below on page 98) specifically returns a flag indicating that

a device is bad, as well as some information about the stage of the

reset/enumeration process where the problem occurred.

After a device has been attached to the bus, powered up, and reset, it

automatically responds to address 0.  As part of the enumeration

process, the host driver attempts to assign the device a "real" address

(something other than zero).  Only one device at a time can respond to

any particular address, including address zero.  Therefore, there can

only be one device at a time on any particular bus in the enumeration

process, and the enumeration process itself can take many seconds to

complete.  While there can be many active (already enumerated) devices

on the bus at the same time (in effect, multitasking), this can only

happen after all of the devices are enumerated one at a time.  If there

are several devices attached to a bus when it is first initialized (when

the host driver is first installed into memory), it may take a long time

(many tens of seconds) for all the devices attached to the bus to get

enumerated.

After a device is attached to the bus and enumerated, it is possible to

have it re-enumerated, either with a physical disconnect and re-

insertion, or with a "soft reset".  After the re-enumeration process is

complete, the device may not necessarily have the same address it did

when it was enumerated the first time.  It is never safe to assume that

a device's address will stay the same across a reset.

BIOS-LEVEL USB API 0.05            Page 25 of 168

POWER DISTRIBUTION

������������������

Power distribution on a USB bus can be the source of all kinds of

consternation.  The USB architecture makes an attempt to manage power on

devices attached to the bus, but unfortunately does not do a very good

job of it.  The problem is not necessarily with the USB architecture

itself, but rather with the way it is implemented and enforced.

There are four wires in a standard USB cable.  Two of the wires carry

the actual USB bus signals, and the other two wires carry power from the

upstream hub(s) to the downstream device(s).  All USB devices require

power to do anything useful.  A bus-powered device draws its power from

the upstream hub (via the two power wires).  A self-powered device has

its own power source, and does not draw any power (or at least does not

draw a significant amount of power) from the bus.  Usually, a self-

powered device is plugged into an AC power outlet, but it could also use

batteries, solar power, or some other type of power source.

The source of power (self-powered or bus-powered) a device uses is

identified in the configuration descriptor of the device, as is the

amount of bus power it draws (if it is bus-powered).  If a device is

self-powered, there should never be a power-related problem when you

hook it up to a bus and try to get it to work.  A bus-powered device, on

the other hand, can cause problems.

A bus-powered device obtains its power from the upstream hub that it is

attached to.  Hubs are like any other USB device, in that they are

either bus-powered or self-powered.  A bus-powered hub draws its power

from its upstream hub, and then also "redistributes" the extra power

that it does not itself use to its downstream devices.  A self-powered

hub can provide up to 500 mA (milliamps) of power to each downstream

port.  Therefore, if the total amount of current required by the

downstream devices attached to any particular port exceeds 500mA, the

port will be overloaded.  An overloaded port can cause large segments of

a bus to fail.

A host driver needs to monitor and track the power distribution on the

bus, and (at least theoretically) is not supposed to configure a device

that would cause a bus power overload.  While that is a noble ideal, it

is not actually practical.  The problem is that some devices report

false information (lies) about their power requirements.  For instance,

I personally own two hubs that claim (in their configuration descriptor)

to be self-powered when they are not.  If a host driver were to base its

power distribution scheme on such false information, it could cause

catastrophic bus failures.  Since at least some USB devices misrepresent

their true power requirements, a host driver may simply choose to ignore

power issues and leave it up to the user to monitor and resolve power-

related issues.

BIOS-LEVEL USB API 0.05            Page 26 of 168

There are some other issues to be aware of related to power distribution

through hubs.  Hubs (the mechanism that distributes power to bus-powered

devices) are not all created equal.  Specifically, there are two areas

regarding power distribution where hubs are allowed to differ.  Those

two areas are power-switching (being able to turn on or off the power to

a specific downstream port), and overcurrent-protection (being able to

sense and react to power overloads on downstream ports).

There are three different options a hub can have regarding power-

switching: per-port-power-switching, global-power-switching, and no-

power-switching.  If a hub has per-port-power-switching, the hub allows

each individual port to have the power turned on or off without

affecting any of the other ports on the hub.  If a hub has global-power-

switching, all of the downstream ports are "ganged" together as far as

power switching is concerned.  That is, there is either power to all of

the ports or there is power to none of the ports - it is not possible to

control power to an individual port without affecting all of the other

ports at the same time.  If a hub has no-power-switching, then there is

always power to all of the ports, and it is not possible to turn the

power off at all.  The only type of hub that is allowed to have no-

power-switching is a root hub -- a "regular" (remote) hub must either

have per-port-power-switching or global-power-switching.

There are certain INT 14h requests that allow a user to control the

power to individual ports on the bus.  Specifically, they are the "Power

On Device Given Device Address", "Power Off Device Given Device

Address", and "Power Reset Device Given Device Address" requests,

described below on pages 90, 92, and 94, respectively.  A host driver

will reject a request to control the power of a port on a hub that does

not have per-port-power-switching.

There are also three different options a hub can have regarding

overcurrent-protection: per-port-overcurrent-protection, global-

overcurrent-protection, and no-overcurrent-protection.  If a hub has

per-port-overcurrent-protection, the hub will not allow more than 500mA

of current to flow out of any individual port.  If a hub has global-

overcurrent-protection, it will not allow the total current flow out of

all of the ports combined to exceed a specified value.  For example, a

self-powered four-Port hub that has global-overcurrent-protection would

not allow more than 2000mA to flow out all of the ports combined.  In

theory, it would be possible for all 2000mA to go out one individual

port, and no current at all to flow out of any of the other three ports.

A hub that has no-overcurrent-protection, as the name implies, has no

protection whatsoever.  It is possible for a hub with no-overcurrent-

protection to literally melt down if there are problems either with the

hub itself or with downstream devices, which could cause all kinds of

misery.

BIOS-LEVEL USB API 0.05            Page 27 of 168

Both the type of power-switching a hub has and the type of overcurrent-

protection it has are detailed in the hub descriptor.  IMHO (In My

Humble Opinion), the USB architecture should have never allowed for

anything except per-port-power-switching and per-port-overcurrent-

protection, but it does not do that.  You may need to be careful about

the types of hubs that you purchase if you want your USB devices to work

correctly and safely.

As stated earlier, while the USB architecture makes an attempt to manage

the power distribution on a bus, it does not do a very good job of it.

There are still many things that are left up to the user to monitor and

troubleshoot, which cannot be handled automatically by the USB hardware

or software.

BIOS-LEVEL USB API 0.05            Page 28 of 168

ROOT HUBS AND DEVICE ADDRESS 1

������������������������������

There is a special entity on a USB bus that is called the root hub.  It

is in some ways similar to a "real" (remote) hub, but in most ways is

completely different.  The root hub is not a USB device that you can

simply pick up and look at.  Physically, it is integrated and

inseparable from the USB host hardware.  A root hub is a logical device,

and provides the same functions that a "real" hub provides (it allows

the physical and logical extension of the USB bus to "real" devices).

The way a root hub physically provides those functions, though, is

COMPLETELY different than the way a "real" hub does it.  It gets even

more convoluted than that, because different types of host hardware (for

example, UHCI vs. OHCI) provide the root hub functionality in completely

different ways.  Under this API, the host driver is responsible for

providing the root hub API functionality, since exactly how a root hub

works is hardware-dependent.

A root hub is not a physical device, and therefore does not respond to

USB requests in the same way that "real" devices do.  However, there are

times when it is either desired or required to refer to the root hub as

if it were a real device.  Therefore, under this API, the root hub is

always assigned a device address.  Specifically, the root hub always has

address 1, while "real" devices are assigned addresses between 2 and

127.  The host driver sets up the root hub to "look like" a real device

with address 1, a single end point (endpoint 0), and a maximum packet

size of 64 bytes.

Even though the root hub is at the "head" of the bus, it is not actually

a device attached to the bus, and therefore does not respond to packets

sent out across the bus.  If a packet directed at device address 1 is

actually sent out across the bus, the packet will always fail.  Believe

it or not, this actually provides an incredibly useful tool when used

appropriately.

Most of the time, a host driver will reject any INT 14h requests that

are directed at non-existent device addresses or end points.  However, a

host driver will allow periodic interrupt packets with address 1 and end

point 0 to be sent across the bus, even though it knows the packets will

fail.  This provides the ability to use the USB bus for timing and

testing purposes.  You can issue a "Schedule Periodic Interrupt

Transaction" request (described on page 137 below) to address 1, end

point 0, and it will work.  Of course, every time the periodic interrupt

occurs, the status code (returned in the AX register) will contain an

error, which can simply be ignored.

BIOS-LEVEL USB API 0.05            Page 29 of 168

Under this API, whenever a device is disconnected from the bus, the host

driver automatically terminates any packet transactions that are

scheduled to go to the device.  The host driver does this by returning a

status of TDStsDvcRemoved, and then unscheduling and deleting the

appropriate transfer descriptors.  A root hub is not a real device, and

can never be disconnected from the bus.  Therefore, the host driver will

never automatically unschedule or delete any of the "fake" packet

transactions scheduled to go to address 1.  Any program that schedules

packet transactions to address 1 must unschedule those transactions

manually when appropriate.  This is done with the "Close Transaction

Handle" request (page 147).

BIOS-LEVEL USB API 0.05            Page 30 of 168

MEMORY ADDRESSES

����������������

This API is written under the assumption that the operating system the

API will be used with is DOS, and that the USB host hardware will be

attached to the computer through a PCI bus.  In today's environment,

that will most certainly be the case.  All other major operating systems

(like the Unix/Linux variants and Microsoft Windows) have been updated

to include native USB support, and there is no USB host hardware being

sold today that attaches to any type of bus other than PCI.  That may no

longer be the case 10 years from now, but it is certainly true today.

There are two different types of memory addresses that are used by this

API: code addresses and data addresses.  Many times, another program

(like a device/interface driver) must provide a code address (in the

form of a call back address) to a USB host driver.  The host driver will

call this code address at various times (exactly how and why a call back

address will be called is discussed in various sections throughout this

document).  When provided, a code address (call back address) must be a

standard segment:offset address in the first megabyte of memory (the

only format and location allowed to be called by DOS programs).

When another program provides the host driver a data address, however,

there are two different possibilities.  The data address can be provided

in a standard segment:offset format, or it can be provided as a 32-bit

physical memory address.  Data is transferred between physical memory

and the USB host hardware through the PCI bus using a process called bus

mastering.  Bus mastering allows the PCI bus to access all of the

physical memory on the computer directly, completely bypassing the CPU.

This allows the USB hardware to directly access parts of memory (like

extended memory) that is normally very difficult (or sometimes even

impossible) to access directly from DOS.

In order to use bus mastering, what must be provided to the USB (PCI)

hardware is ALWAYS a physical memory address -- it can never be a

segment:offset address.  If a host driver receives a segment:offset

address as part of a data transfer request (which it almost always will

in a DOS environment), it must convert the segment:offset address to a

physical address as required by the host hardware.  There are certain

situations where this is possible, and others where it is not.

BIOS-LEVEL USB API 0.05            Page 31 of 168

A host driver can easily perform the conversion if the data address is

in conventional memory (in the first 640 kB).  The conversion can also

be performed if the data address is in an Upper Memory Block (UMB,

between 640 kB and 1 MB), as long as the CPU is running in protected

mode under a VCPI-compliant memory manager (like EMM386).  The

conversion cannot be made under any other protected mode environment

(such as Windows DPMI).  In addition, UMB's as provided by a program

like UMBPCI cannot be used (even though the CPU is in real mode),

because the memory used by UMBPCI to create UMB's is not physical memory

that can be accessed though PCI bus mastering.  In an environment where

direct memory address conversion cannot be performed (such as DPMI or

UMB's provided by UMBPCI), it is sometimes possible to design a double-

buffering process, though that functionality is not provided by any

existing program.

BIOS-LEVEL USB API 0.05            Page 32 of 168

INT 14h API EXTENSIONS

����������������������

This API is implemented as an extension to the BIOS serial services

interrupt (INT 14h).  There are currently only two functions provided at

the INT 14h level: an install check (AX = 5000h) and a transaction

request (AX = 5001h).  Additional functions will be required in the

future, particularly as the API is expanded to include support for USB

2.0 (EHCI), 2.5 (WHCI), and 3.0 (XHCI) functionality.  The INT 14h

request is issued with appropriate values (as detailed directly below)

in the AX, BX, CX, DX, and/or DS registers.  The request returns with

the carry flag indicating success or error (CF = clear if success), AX

containing an error code (the error codes are listed below on page 35),

and request-specific data in the BX, CX, and/or DX registers.  In

addition, the ES register may be used to return request-specific data in

future versions of the API, though it is not currently used.

Install Check

�������������

Entry:

AX = 5000h

BX = 5553h ("US")

CX = 4221h ("B!")

Return (Success):

CF = Clear

AX = 0

BX = 4221h ("B!")

CX = 5553h ("US")

Return (Error):

Since the API is not installed, it cannot control how the BIOS

will respond to an INT 14h.  However, the BIOS will usually return

with registers and flags unchanged.  Pre-setting the carry flag

before the request can help ease the identification process.

Notes: If there is at least one host driver installed in memory,

this call will return successfully.  No matter how many host

drivers are actually installed, the first host driver to

receive the INT 14h request will respond in the affirmative.

BIOS-LEVEL USB API 0.05            Page 33 of 168

Transaction Request

�������������������

Entry:

AX = 5001h

BX = 5553h ("US")

CX = 4221h ("B!")

DS:[DX] = Pointer to 64-byte request structure (see below)

Return (Success):

CF = Clear

AX = 0

BX, CX, DX = Depends on request type

Return (Error):

CF = Set

AX = Error code (see error code table on page 35)

BX = CX = DX = FFFFh

Notes: This particular request actually hosts a multitude of

different types of sub-requests.  Details about a specific

request are contained in the 64-byte request data structure

pointed at by the DS:[DX] register combination.  The 64-byte

request structure is detailed in the next section, "THE 64-

BYTE REQUEST STRUCTURE" (page 37).

BIOS-LEVEL USB API 0.05            Page 34 of 168

ERROR CODES

(Returned immediately by all INT 14h Requests)

Part 1 of 2

����������������������������������������������������������������ͻ

�     Mnemonic        �Value�             Description            �

����������������������������������������������������������������͹

�I14RErrSuccess       �0000h�No Error at all (Success!)          �

����������������������������������������������������������������Ķ

�I14RErrResources     �0001h�Insufficient Resources/Memory       �

�I14RErrHostHardware  �0002h�Unknown Hardware Problem with Host  �

�I14RErrReqTypeNoSupt �0003h�Request Type Not Supported          �

����������������������������������������������������������������Ķ

�I14RErrRequestType   �0011h�Invalid Request Type                �

�I14RErrFlags         �0012h�Invalid Flags                       �

�I14RErrHostIndex     �0013h�Invalid Host Index                  �

�I14RErrDeviceAddress �0014h�Invalid Device Address              �

�I14RErrHubAddress    �0014h�Invalid Hub Address                 �

�I14RErrEndPoint      �0015h�Invalid End Point                   �

�I14RErrHubPort       �0015h�Invalid Hub Port                    �

�I14RErrAltIntf       �0015h�Invalid Alternate Interface         �

�I14RErrConfigValue   �0016h�Invalid Configuration Value         �

�I14RErrCloseID       �0016h�Invalid Closure ID                  �

�I14RErrInterfaceNum  �0017h�Invalid Interface Number            �

�I14RErrSearchIndex   �0018h�Invalid Search Index                �

�I14RErrVendorProdID  �0019h�Invalid Vendor/Product ID           �

�I14RErrDvcIntf       �001Ah�Invalid Device/Interface Value      �

�I14RErrRequestHandle �001Bh�Invalid Request Handle              �

�I14RErrPeriodicity   �001Ch�Invalid Periodicity                 �

�I14RErrBeepFrequency �001Ch�Invalid Beep Frequency              �

�I14RErrTimeOut       �001Dh�Invalid Timeout                     �

�I14RErrDataAddress   �001Eh�Invalid Data Address                �

�I14RErrDataSize      �001Fh�Invalid Data Size                   �

�I14RErrCallBackAddr  �0021h�Invalid Call Back Address           �

�I14RErrLCRtnCode     �0021h�Invalid Large Call Return Code      �

�I14RErrUserPktID     �0022h�Invalid User Packet ID              �

�I14RErrSetupReqData  �0023h�Invalid Setup Request Data          �

�I14RErrFrameTiming   �0024h�Invalid Frame Timing                �

�I14RErrFrameIndex    �0024h�Invalid Frame Index                 �

�I14RErrIsochArray    �0025h�Invalid Entry in Isoch Array Data   �

����������������������������������������������������������������ͼ

BIOS-LEVEL USB API 0.05            Page 35 of 168

ERROR CODES

(Returned immediately by all INT 14h Requests)

Part 2 of 2

����������������������������������������������������������������ͻ

�     Mnemonic        �Value�             Description            �

����������������������������������������������������������������͹

�I14RErrHostHalted    �0031h�Host is Halted/Suspended            �

�I14RErrIntfNotFound  �0032h�Interface Not Found                 �

�I14RErrIntfInConfig  �0033h�Interface Already Being Configured  �

�I14RErrIntfAlreadyReg�0034h�Interface Already Registered        �

�I14RErrHostCallInUse �0035h�Host Run/Stop/etc. Already Running  �

�I14RErrNoPortPower   �0036h�Per-Port-Power not Supported by Hub �

�I14RErrAlreadyBeeping�0037h�Beep Request Already Running        �

����������������������������������������������������������������Ķ

�I14RErrDebugMode     �0041h�Tried Single-Step with no Debug Mode�

����������������������������������������������������������������Ķ

�I14RErrTimingOwned   �0051h�Problem with Frame Timing Ownership �

�I14RErrTimingLimit   �0052h�Frame Timing Already at a Limit     �

�I14RErrTmgChgInProg  �0053h�Timing Change Already in Progress   �

����������������������������������������������������������������ͼ

BIOS-LEVEL USB API 0.05            Page 36 of 168

THE 64-BYTE REQUEST STRUCTURE

�����������������������������

As part of a INT 14h USB transaction request (AX = 5001h, discussed in

the previous section), there is provided a pointer to a 64-byte request

structure.  The 64-byte structure contains details on the exact request

type, as well as any "extra" data that may be required to complete the

transaction.

The basic detail of a 64-byte Int 14h request structure is given below,

in the format of an A86 assembly language structure.  The first byte of

the structure is the request type (I14RRequestType), which is the

"cornerstone" of the rest of the structure.  That is, exactly which

other fields in the structure may or may not need to be filled in, and

exactly what the those fields mean, depends on what is in the

I14RRequestType field.

The request types are divided into different classes.  For example,

there is a host class (00h), timing class (10h), hub class (20h), etc.

The class identifiers (which all end with a 0h) are not legal request

types -- they are simply used to help organize things.  For example, if

you actually tried to send a request with an I14RRequestType of 10h

("Frame Timing Class"), you would receive an error.

When you request a transaction with the host driver, the driver will

immediately return with the carry flag and an error code in AX

indicating success or error (CF = clear and AX = 0 is success, CF = set

and AX > 0 is an error).  The exact error codes are detailed in the

error code table above on page 35.  Depending on the specific request

type, it may also return values in BX, CX, and/or DX.  In some cases,

that will entirely complete the transaction.  In other cases, it will

cause transaction(s) to be scheduled on the USB bus (see the "IMMEDIATE,

SCHEDULE, AND LARGE TRANSACTIONS" section above on page 11).

Immediately after the transaction request returns, the 64-byte request

structure can be reused to issue another transaction request (if so

desired).  While processing the transaction request, the host driver

"copies" all of the relevant data it needs (addresses, codes, etc.) from

the request structure and "stores" it until the entire transaction is

completed (which may be a long time after the transaction request has

returned).  Note, however, that the transaction request only "stores"

the relevant data that is contained in the request structure itself --

it will NOT store/copy/buffer any data outside the request structure

(for example, data stored in memory with the memory address pointer

stored in the request structure).

BIOS-LEVEL USB API 0.05            Page 37 of 168

Each individual request type is explained in great detail in the section

below, "TRANSACTION REQUEST TYPES" (page 42).  If a particular field of

the structure is not listed in the description for that particular

request type, the field will be ignored.  That is, the field can contain

anything and it will not affect the status or result of the request.

However, the 14 "reserved" bytes at the end of the structure must be set

to zero.

BIOS-LEVEL USB API 0.05            Page 38 of 168

Int14RequestSize EQU 64 ;Size of Request Structure

Int14RequestStruc         STRUC      ;Structure for an Int 14h Request

I14RRequestType            DB   ?  ;Type of Request

I14RRTHostClass         EQU  00h ;Host/System/OS Class

I14RRTGetHostSWInfo   EQU  01h ;Get Host Software Info

I14RRTGetHostHWInfo   EQU  02h ;Get Host Hardware Info

I14RRTGetHostVendInfo EQU  03h ;Get Host Vendor Info

I14RRTGetHostStatus   EQU  04h ;Get Current Host Status

I14RRTHostRun         EQU  08h ;Start/Run/Resume the Host

I14RRTHostStop        EQU  09h ;Stop the Host

I14RRTHostReset       EQU  0Ah ;Reset the Host

I14RRTHostSuspend     EQU  0Bh ;Global Suspend the Host

I14RRTHostResume      EQU  0Ch ;Force Global Resume on Host

I14RRTTimingClass       EQU  10h ;Frame Timing Class

I14RRTRegTmgOwner     EQU  11h ;Register as Timing Owner

I14RRTUnRegTmgOwner   EQU  12h ;UnRegister as Timing Owner

I14RRTIncTiming       EQU  13h ;Increment (Slow Down) Frame Tmg

I14RRTDecTiming       EQU  14h ;Decrement (Speed Up) Frame Tmg

I14RRTChangeTiming    EQU  15h ;Change Frame Timing (Large Amt)

I14RRTHubClass          EQU  20h ;Hub Class

I14RRTGetDvcHubInfo   EQU  21h ;Get Hub Info for Device

I14RRTNewDvcConn      EQU  24h ;Device has been connected

I14RRTDvcDisc         EQU  25h ;Device has been disconnected

I14RRTSendHubChar     EQU  27h ;Send Hub Characteristics to Host

I14RRTEnableHubPort   EQU  28h ;Enable Device given Hub & Port

I14RRTDisableHubPort  EQU  29h ;Disable Device given Hub & Port

I14RRTResetHubPort    EQU  2Ah ;Reset Device given Hub & Port

I14RRTSuspendHubPort  EQU  2Bh ;Suspend Device given Hub & Port

I14RRTResumeHubPort   EQU  2Ch ;Resume Device given Hub & Port

I14RRTPwrOnHubPort    EQU  2Dh ;Power On Device given Hub & Port

I14RRTPwrOffHubPort   EQU  2Eh ;Power Off Dvc given Hub & Port

I14RRTPwrResetHubPort EQU  2Fh ;Power Reset Dvc given Hub & Port

I14RRTTPowerClass       EQU  30h ;Power Class

I14RRTGetDvcPowerInfo EQU  31h ;Get Power Info for Device

I14RRTGetHubPowerDraw EQU  32h ;Get Power Draw Info for Hub

I14RRTPwrOnDevice     EQU  3Dh ;Power On Device given Dvc Addr

I14RRTPwrOffDevice    EQU  3Eh ;Power Off Device given Dvc Addr

I14RRTPwrResetDevice  EQU  3Fh ;Power Reset Dvc given Dvc Addr

I14RRTDeviceClass       EQU  40h ;Device Class

I14RRTGetDvcClassInfo EQU  41h ;Get Device Class Info

I14RRTGetDvcVendInfo  EQU  42h ;Get Device Vendor Info

I14RRTGetDvcStatus    EQU  43h ;Get Device Status

I14RRTEnableDevice    EQU  48h ;Enable/Resume Dvc given Dvc Addr

I14RRTDisableDevice   EQU  49h ;Disble Device given Dvc Addr

I14RRTResetDevice     EQU  4Ah ;Reset Device given Dvc Addr

I14RRTSuspendDevice   EQU  4Bh ;Suspend Device given Dvc Addr

I14RRTResumeDevice    EQU  4Ch ;Resume Device given Dvc Addr

BIOS-LEVEL USB API 0.05            Page 39 of 168

I14RRTConfigClass       EQU  50h ;Configuration Class

I14RRTConfigingIntf   EQU  51h ;Interface is Being Configured

I14RRTConfigIntfDone  EQU  52h ;Interface Configuration is Done

I14RRTSetNewConfig    EQU  58h ;Set/Change Device Config Value

I14RRTInterfaceClass    EQU  60h ;Interface Class

I14RRTFindRegIntf     EQU  62h ;Look for Registered Interface

I14RRTFindUnRegIntf   EQU  63h ;Look for Unregistered Interface

I14RRTRegIntfOwner    EQU  64h ;Register as Interface Owner

I14RRTUnRegIntfOwner  EQU  65h ;Unregister as Interface Owner

I14RRTIntfDontLook    EQU  68h ;Existing Intf Owner Don't Look

I14RRTAltIntfClass      EQU  70h ;Alternate Interface Class

I14RRTGetAltIntfInfo  EQU  71h ;Get Alternate Interface Info

I14RRTEndPointClass     EQU  80h ;End Point Class

I14RRTGetDataX        EQU  81h ;Get Current DataX Value

I14RRTIncDataX        EQU  88h ;Increment/Toggle DataX Value

I14RRTPacketClass       EQU  90h ;Packet Class

I14RRTDoIsoch         EQU  94h ;Schedule Isochronous Transaction

I14RRTDoInterruptPer  EQU  95h ;Schedule Periodic Interrupt

I14RRTDoControl       EQU  96h ;Schedule Control/Setup Request

I14RRTDoBulk          EQU  97h ;Schedule Bulk Transaction

I14RRTDoInterrupt1T   EQU  98h ;Schedule One-Time Interrupt

I14RRTCloseHandle     EQU  9Ch ;Close Scheduled Transaction

I14RRTChangeIntPer    EQU  9Dh ;Change Periodicity of Interrupt

I14RRTGetTransStatus  EQU  9Fh ;Get Status of Packet Transaction

I14RRTMiscClass         EQU 0A0h ;Miscellaneous Class

I14RRTLargeCallDone   EQU 0A1h ;Large Request Completed

I14RRTBeepSpeaker     EQU 0AFh ;Beep the Speaker

I14RRTInterHostClass    EQU 0E0h ;Inter-Host Communication Class

I14RRTHostAdded       EQU 0E1h ;New Host Driver installed

I14RRTHostDeleted     EQU 0E2h ;Host Driver removed

I14RRTRegDvc0Host     EQU 0E3h ;Device0 Registration

I14RRTUnRegDvc0Host   EQU 0E4h ;Device0 UnRegistration

I14RRTCopyNDTable     EQU 0E5h ;Copy New Device Ownership Table

I14RRTInternalClass     EQU 0F0h ;Host Internal/Troubleshooting

I14RRTHostDebug       EQU 0FDh ;Put Host Into Debug Mode

I14RRTHostSingleStep  EQU 0FEh ;SingleStep the Host (Debug Mode)

I14RFlags                  DB   ?  ;Bit-level flags

I14RFlagIn              EQU  01h ;In Direction

I14RFlagLowSpeed        EQU  02h ;Low-Speed Device

I14RFlagHiSpeed         EQU  04h ;High-Speed Device

I14RFlagNoRetries       EQU  10h ;No Auto-Retries for Control Pkts

I14RFlagShortPktOK      EQU  20h ;No Auto-Retry for Short Pkt Err

I14RFlagSpecificFrame   EQU  40h ;Use Specific Frame Number

I14RFlagAddrIsPhys      EQU  80h ;Data Address is Physical

I14RHostIndex              DB   ?  ;Host Host Index

I14RDeviceAddress          DB   ?  ;USB Device Address

I14RHubAddress          EQU (OFFSET I14RDeviceAddress)

I14REndPoint               DB   ?  ;EndPoint or Port or Alt Intf

I14RHubPort             EQU (OFFSET I14REndPoint)

I14RAltInterface        EQU (OFFSET I14REndPoint)

BIOS-LEVEL USB API 0.05            Page 40 of 168

I14RConfigValue            DB   ?  ;Configuration Value

I14RCloseID             EQU (OFFSET I14RConfigValue)

I14RDataX               EQU (OFFSET I14RConfigValue)

I14RInterfaceNum           DB   ?  ;Interface Number

I14RSearchIndex            DB   ?  ;Search Index

I14RVendorID               DW   ?  ;Vendor ID

I14RProductID              DW   ?  ;Product ID

I14RDvcClass               DB   ?  ;Device Class

I14RDvcSubClass            DB   ?  ;Device SubClass

I14RDvcProtocol            DB   ?  ;Device Protocol

I14RIntfClass              DB   ?  ;Interface Class

I14RIntfSubClass           DB   ?  ;Interface SubClass

I14RIntfProtocol           DB   ?  ;Interface Protocol

I14RRequestHandle          DW   ?  ;Request Handle Number

I14RPeriodicity            DW   ?  ;Interrupt Periodicity/Duration

I14RBeepFrequency       EQU (OFFSET I14RPeriodicity)

I14RTimeout                DW   ?  ;Transaction Time Out Value

I14RDataAddress            DD   ?  ;Data Address

I14RDataSize               DW   ?  ;Size of Data (Bytes)

I14RCallBackAddr           DD   ?  ;Call Back Address (CS:IP format)

I14RLargeCallRtnCode    EQU (OFFSET I14RCallBackAddr)

I14RUserPktID              DW   ?  ;User Packet ID

I14RSetupReqData           DQ   ?  ;Setup Request Data (8 bytes)

I14RFrameTiming            DW   ?  ;Frame Timing Value (def = 12000)

I14RFrameIndex          EQU (OFFSET I14RFrameTiming)

;Starting Frame of Isoch Schedule

I14RIsochSchedAddr         DD   ?  ;Address of Isoch Schedule

I14RFiller DB (Int14RequestSize-$) DUP (0) ;Reserved/Future Use

ENDS

BIOS-LEVEL USB API 0.05            Page 41 of 168

TRANSACTION REQUEST TYPES

�������������������������

As discussed in the "INT 14h API EXTENSIONS" section above (page 33), a

specific API transaction is initiated by issuing an INT 14h with AX =

5001h, BX = 5553h ("US"), CX = 4221h ("B!"), and DS:[DX] pointing at a

64-byte data structure.  The exact format of the 64-byte data structure

is discussed in the previous section, "THE 64-BYTE REQUEST STRUCTURE"

(page 37).

There are several data fields that are detailed in the data structure,

but only two of them are required for every transaction: I14RRequestType

(a byte at offset 0) and I14RHostIndex (a byte at offset 2).  Other

fields in the structure may also be required, depending on the exact

request type (determined by the combination of the contents in

I14RRequestType and I14RHostIndex).  Below are listed the details of

each request type, including which fields in the data structure are

required for each different request type, and exactly what those fields

mean.  If a particular field is not listed as pertinent to a particular

request type, it is ignored.  That is, it can contain anything at all

when the transaction request is issued, and it will not affect the

results.

When the INT 14h request returns, it will have the carry flag indicating

success or failure (carry flag clear = success), an error code in AX (AX

= 0 if success), and transaction-specific data in BX, CX, DX, and (in

future versions of the API) ES.  Exact details on what is returned in

BX, CX, and DX are given below in the description for each request type.

The error codes returned in AX are detailed above on page 35.

In the details for each request there is also indicated a return type,

which can be immediate, schedule, or large.  Immediate transactions are

completed as soon as the INT 14h request returns, while schedule and

large transactions schedule background process to occur which will be

completed at some other (usually much later) point in time.  Exact

details on the differences between the three different return types are

discussed in the "IMMEDIATE, SCHEDULE, AND LARGE TRANSACTIONS" section

above (page 11).

BIOS-LEVEL USB API 0.05            Page 42 of 168

Get Host Software Information

�����������������������������

Class:       Host

Return Type: Immediate

Entry:

I14RRequestType = 01h (I14RRTGetHostSWInfo)

I14RHostIndex   = Host Index (0-15, 255 = All Hosts)

Return (Success, I14RHostIndex = 255 (All Hosts)):

CF = Clear

AX = 0

BX = Host Driver API Version (BCD)

BH = Major Version

BL = Minor Version

e.g., version 0.05 = 0005h

CX = FFFFh

DX = FFFFh

Return (Success, 0 <= I14RHostIndex <= 15):

CF = Clear

AX = 0

BL = USB Version (BCD)

High Nibble = Major Version

Low Nibble  = Minor Version

e.g., version 1.0 = 10h

BH = Int 2Fh (TSR Multiplex) Handle Number (C0h-FFh)

If Host Driver does not use Int 2Fh, CH = 0

CX = FFFFh

DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: If this request is issued with I14RHostIndex = 255 (all

hosts), the first host driver to receive the request will

process the request.  It will not "pass on" the request to

other host driver(s).

All Host Drivers installed in memory at the same time must

support the same version of the Driver API.

BIOS-LEVEL USB API 0.05            Page 43 of 168

Get Host Hardware Information

�����������������������������

Class:       Host

Return Type: Immediate

Entry:

I14RRequestType = 02h (I14RRTGetHostHWInfo)

I14RHostIndex   = Host Index (0-15, 255 = Any Host)

Return (Success, I14RHostIndex = 255 (Any Host)):

CF = Clear

AX = 0

BX = Installed Hosts Bitmap

Bit 0 = Set if Host Index 0 is Installed, etc.

CX = FFFFh

DX = FFFFh

Return (Success, 0 <= I14RHostIndex <= 15):

CF = Clear

AX = 0

BL = Host Type and Bus Type (not a Bitmap)

Low Nibble = Host Type:

01h = UHCI

02h = OHCI

03h = EHCI

04h = WHCI

05h = XHCI

High Nibble = Bus Type:

10h = PCI

BH = FFh

CL = IRQ Line Number (3-15)

CH = Number of Root Hub Ports

DX = PCI Bus-Device-Function Word

Bits 0-2  = Function (0-7)

Bits 3-7  = Device (0-31)

Bits 8-15 = Bus (0-255)

If Host is not PCI-attached, DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: A UHCI host always reports as having exactly 2 ports, even

though both ports may not physically be present.

The format of the PCI bus-device-function word is in the same

format that it is received from the PCI BIOS.

BIOS-LEVEL USB API 0.05            Page 44 of 168

If this request is issued with I14RHostIndex = 255 (any

host), the first host driver to receive the request will

process the request.  It will not "pass on" the request to

other host driver(s).

BIOS-LEVEL USB API 0.05            Page 45 of 168

Get Host Vendor Information

���������������������������

Class:       Host

Return Type: Immediate

Entry:

I14RRequestType   = 03h (I14RRTGetHostVendInfo)

I14RHostIndex     = Host Index (0-15)

Return (Success):

CF = Clear

AX = 0

BX = Host Controller Vendor ID

CX = Host Controller Product ID

DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This returns the PCI vendor ID and product ID as stored in

the PCI configuration space for the host controller.  This is

the same information returned by a "Get Device Vendor

Information" request (page 97) issued for device address 1

(the root hub).

BIOS-LEVEL USB API 0.05            Page 46 of 168

Get Host Status

���������������

Class:       Host

Return Type: Immediate

Entry:

I14RRequestType = 04h (I14RRTGetHostStatus)

I14RHostIndex   = Host Index (0-15)

Return (Success):

CF = Clear

AX = 0

BL = Host Status

1 = Running normally

2 = Stopped/Halted

3 = Global Suspend Mode

4 = Debug (Single-Step) Mode

255 = Unknown/Undefined

BH = Percentage of Bus Bandwidth currently in use

CX = Current Frame Index (0-1023)

DX = Frame Timing (Full-Speed Bit-Times/Frame, default = 12000)

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: According to the USB specification, 90% of the bus bandwidth

is "reserved" for isochronous and interrupt transactions, and

the remaining 10% is "left over" for control and bulk

transactions.  The value returned in BH is how much of the

90% isochronous/periodic interrupt bandwidth is actually

being used (NOT how much of the entire bus is being used).

This is a fairly good approximation of how "full" the bus

currently is.

BIOS-LEVEL USB API 0.05            Page 47 of 168

Host Run

��������

Class:       Host

Return Type: Large

Entry:

I14RRequestType  = 08h (I14RRTHostRun)

I14RHostIndex    = Host Index (0-15, 255 = All Hosts)

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

AX = 0

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Owner Notifications (Success):

AX = 0018h (OwnerCallHostRun)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = DX = 0

Notifies interface owners of all devices attached to the

host(s), all device 0 owners, and the frame timing owner.

Notes: This request will do whatever it takes to attempt to run the

host(s) "normally" again, no matter what state(s) the host(s)

are currently in.  I.e., this request will "recover" a host

that is simply halted, one that is in debug mode, or one that

is currently in global suspend mode.

BIOS-LEVEL USB API 0.05            Page 48 of 168

If this request is issued with I14RHostIndex = 255 (all

hosts), the first host driver to receive the request will

"pass on" the request to all host drivers individually.

BIOS-LEVEL USB API 0.05            Page 49 of 168

Host Stop

���������

Class:       Host

Return Type: Large

Entry:

I14RRequestType  = 09h (I14RRTHostStop)

I14RHostIndex    = Host Index (0-15, 255 = All Hosts)

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

AX = 0

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Owner Notifications (Success):

AX = 0019h (OwnerCallHostStop)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = DX = 0

Notifies interface owners of all devices attached to the

host(s), all device 0 owners, and the frame timing owner.

Notes: This request simply stops the host from generating timing on

the bus.  It does not unschedule or reschedule any bus

transactions.  After the bus is started again, any pending

transactions will be processed as originally scheduled.

If this request is issued with I14RHostIndex = 255 (all

hosts), the first host driver to receive the request will

"pass on" the request to all host drivers individually.

BIOS-LEVEL USB API 0.05            Page 50 of 168

Host Reset

����������

Class:       Host

Return Type: Large

Entry:

I14RRequestType  = 0Ah (I14RRTHostReset)

I14RHostIndex    = Host Index (0-15, 255 = All Hosts)

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

AX = 0

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Owner Notifications (Success):

AX = 001Ah (OwnerCallHostReset)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = DX = 0

Notifies interface owners of all devices attached to the

host(s), all device 0 owners, and the frame timing owner.

In addition to the "Host Reset" owner call, ALL devices attached

to the host, either directly or through a remote hub, will be

disconnected.  This will result in device disconnect

notifications to all interface owners of devices attached to the

host.  Because owner(s) will be notified of each individual

device being disconnected, there is typically no need for an

interface owner to do anything directly in response to a host

reset notification.

BIOS-LEVEL USB API 0.05            Page 51 of 168

Notes: This request performs a complete (global) reset of the

host(s).  All "real" devices attached to the host, either

directly or indirectly, will effectively be disconnected.

All scheduled bus transactions to those devices are

automatically deleted (unscheduled).  Owners MUST NOT issue

"Close Handle" requests to unschedule the requests.

The process that occurs after the host reset is complete is

essentially the same as the process that occurs when the host

driver is first installed into memory and begins to control

the host hardware.  All devices that remain attached to the

bus during and after the host reset process will be

automatically re-enumerated as appropriate.  However, the

device address(es) assigned after the re-enumeration process

is complete may not be the same as they were before the

initiation of the host reset process.

Device address 1 (the root hub) is not disconnected during a

host reset.  Therefore, any "fake" (timing-only) transactions

to device address 1 will remain in the schedule, and will

continue to be processed after the reset process is complete

(the host reset process typically takes several hundred

milliseconds to complete).

Frame timing and frame sequence numbers are preserved across

a "Host Reset" request.  Therefore, the frame timing owner

should not need to worry about timing resynchronization, and

the timing-only transactions should resume just as they did

before the host reset was issued.

The large return does not necessarily occur at the end of the

reset process, but rather may occur sometime during the

middle of the process.  The host reset process may continue

for several hundred milliseconds after the large return is

issued.

If this request is issued with I14RHostIndex = 255 (all

hosts), the first host driver to receive the request will

"pass on" the request to all host drivers individually.

BIOS-LEVEL USB API 0.05            Page 52 of 168

Host Suspend (Global Suspend)

�����������������������������

Class:       Host

Return Type: Large

Entry:

I14RRequestType  = 0Bh (I14RRTHostSuspend)

I14RHostIndex    = Host Index (0-15, 255 = All Hosts)

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

AX = 0

BX = I14RUserPktID of Requester

CX= DX = FFFFh

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Owner Notifications (Success):

AX = 001Bh (OwnerCallHostSuspend)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = DX = 0

Notifies interface owners of all devices attached to the

host(s), all device 0 owners, and the frame timing owner.

Notes: This request has the host(s) issue a global suspend signal to

the bus.  To "recover" from a host suspend, the "Host Run"

request (page 48) should be issued, rather than the "Host

Resume" request (page 55).  A "Host Resume" issues a global

resume signal down the bus, but does NOT cause the host to

begin running "normally" again.  The "Host Run" request will

do both.

BIOS-LEVEL USB API 0.05            Page 53 of 168

If this request is issued with I14RHostIndex = 255 (all

hosts), the first host driver to receive the request will

"pass on" the request to all host drivers individually.

BIOS-LEVEL USB API 0.05            Page 54 of 168

Host Resume (Global Resume)

���������������������������

Class:       Host

Return Type: Large

Entry:

I14RRequestType  = 0Ch (I14RRTHostResume)

I14RHostIndex    = Host Index (0-15, 255 = All Hosts)

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

AX = 0

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Owner Notifications (Success):

AX = 001Ch (OwnerCallHostResume)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = DX = 0

Notifies interface owners of all devices attached to the

host(s), all device 0 owners, and the frame timing owner.

Notes: This request has the host(s) issue a global suspend to the

bus, which will only partially "recover" the bus from a "Host

Suspend" request (page 53).  To have the bus begin running

"normally" again, the "Host Run" request (page 48) should be

issued instead of this "Host Resume" request.

BIOS-LEVEL USB API 0.05            Page 55 of 168

The large return does not necessarily occur at the end of the

resume process, but rather may occur sometime during the

middle of the process.  The resume process may continue for

several hundred milliseconds after the large return is

issued.

If this request is issued with I14RHostIndex = 255 (all

hosts), the first host driver to receive the request will

"pass on" the request to all host drivers individually.

BIOS-LEVEL USB API 0.05            Page 56 of 168

Register As Frame Timing Owner

������������������������������

Class:       Timing

Return Type: Immediate

Entry:

I14RRequestType  = 11h (I14RRTRegTmgOwner)

I14RHostIndex    = Host Index (0-15)

I14RCallBackAddr = Notification Call Back Address (CS:IP)

(Frame Timing Owner)

I14RUserPktID    = Value to return in BX on Notifications

Return (Success):

CF = Clear

AX = 0

CX = Frame Timing Owner Handle

BX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Owner Notifications:

AX = (see NOTIFICATION CODES Table on page 10)

BX = I14RUserPktID of Frame Timing Owner

CX = Depends on Notification Code

DX = Depends on Notification Code

The frame timing owner is notified when "changes" occur

somewhere on the bus that would be of interest to the frame

timing owner.  For example, an frame timing owner is notified if

the host is stopped (halted) or reset.  Notifications detailed

in the NOTIFICATION CODES table (page 10) that are related to

the host, as opposed to a device, would be the ones issued to a

frame timing owner.

Notes: The frame timing cannot be changed unless this request has

been issued first, and only the registered frame timing owner

is allowed to make changes to the frame timing.  In addition,

there can be only one registered owner of the frame timing.

The frame timing owner handle (the value returned in CX) will

be a randomly-generated number, and must be "returned" (in

I14RRequestHandle) in all consequent requests related to

frame timing.  The handle number is how the host driver

verifies that a request is being generated by the legitimate

frame timing owner.

BIOS-LEVEL USB API 0.05            Page 57 of 168

The requester will remain the frame timing owner until the

host driver receives an "Unregister as Frame Timing Owner"

request (page 59), or until the host driver is removed from

memory.

BIOS-LEVEL USB API 0.05            Page 58 of 168

Unregister as Frame Timing Owner

��������������������������������

Class:       Timing

Return Type: Immediate

Entry:

I14RRequestType   = 12h (I14RRTUnRegTmgOwner)

I14RHostIndex     = Host Index (0-15)

I14RRequestHandle = Frame Timing Owner Handle

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: The frame timing owner handle is the value that was returned

in the CX register during the "Register as Frame Timing

Owner" request (page 57).

BIOS-LEVEL USB API 0.05            Page 59 of 168

Increment Frame Timing Factor (Slow Down Bus)

���������������������������������������������

Class:       Timing

Return Type: Schedule

Entry:

I14RRequestType   = 13h (I14RRTIncTiming)

I14RHostIndex     = Host Index (0-15)

I14RRequestHandle = Frame Timing Owner Handle

I14RCallBackAddr  = Schedule Return Address (CS:IP)

I14RUserPktID     = Value to return in BX on Schedule Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Schedule Return (Success):

ZF = Set

AX = 0

BX = I14RUserPktID of Requester

CX = New Frame Timing Factor

DX = FFFFh

Schedule Return (Error):

ZF = Clear

AX = Status Code (see Status Code Table on page 14)

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Owner Notifications (Success):

AX = 0021h (OwnerCallTimingChg)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = FFh

DX = New Frame Timing Factor

UHCI Hosts: 11936-12063

Notifies interface owners of all devices attached to the

host(s), all device 0 owners, and the frame timing owner.

BIOS-LEVEL USB API 0.05            Page 60 of 168

Notes: The frame timing owner handle is the value that was returned

in the CX register during the "Register as Frame Timing

Owner" request (page 57).

If the bus is actively running, the host driver will

automatically wait 6 milliseconds before actually changing

the frame timing factor.  This is done to ensure compliance

with the USB specification regarding how quickly a frame

timing factor can be changed.

BIOS-LEVEL USB API 0.05            Page 61 of 168

Decrement Frame Timing Factor (Speed Up Bus)

��������������������������������������������

Class:       Timing

Return Type: Schedule

Entry:

I14RRequestType   = 14h (I14RRTDecTiming)

I14RHostIndex     = Host Index (0-15)

I14RRequestHandle = Frame Timing Owner Handle

I14RCallBackAddr  = Schedule Return Address (CS:IP)

I14RUserPktID     = Value to return in BX on Schedule Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Schedule Return (Success):

ZF = Set

AX = 0

BX = I14RUserPktID of Requester

CX = New Frame Timing Factor

DX = FFFFh

Schedule Return (Error):

ZF = Clear

AX = Status Code (see Status Code Table on page 14)

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Owner Notifications (Success):

AX = 0021h (OwnerCallTimingChg)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = FFh

DX = New Frame Timing Factor

UHCI Hosts: 11936-12063

Notifies interface owners of all devices attached to the

host(s), all device 0 owners, and the frame timing owner.

BIOS-LEVEL USB API 0.05            Page 62 of 168

Notes: The frame timing owner handle is the value that was returned

in the CX register during the "Register as Frame Timing

Owner" request (page 57).

If the bus is actively running, the host driver will

automatically wait 6 milliseconds before actually changing

the frame timing factor.  This is done to ensure compliance

with the USB specification regarding how quickly a frame

timing factor can be changed.

BIOS-LEVEL USB API 0.05            Page 63 of 168

Change Frame Timing Factor (By a Large Amount)

����������������������������������������������

Class:       Timing

Return Type: Schedule

Entry:

I14RRequestType   = 15h (I14RRTChangeTiming)

I14RHostIndex     = Host Index (0-15)

I14RRequestHandle = Frame Timing Owner Handle

I14RCallBackAddr  = Schedule Return Address (CS:IP)

I14RUserPktID     = Value to return in BX on Schedule Return

I14RFrameTiming   = New Frame Timing Factor

UHCI Hosts: 11936-12063

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Schedule Return (Success):

ZF = Set

AX = 0

BX = I14RUserPktID of Requester

CX = New Frame Timing Factor

DX = FFFFh

Schedule Return (Error):

ZF = Clear

AX = Status Code (see Status Code Table on page 14)

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Owner Notifications (Success):

AX = 0021h (OwnerCallTimingChg)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = FFh

DX = New Frame Timing Factor

UHCI Hosts: 11936-12063

Notifies interface owners of all devices attached to the

host(s), all device 0 owners, and the frame timing owner.

BIOS-LEVEL USB API 0.05            Page 64 of 168

Notes: The frame timing owner handle is the value that was returned

in the CX register during the "Register as Frame Timing

Owner" request (page 57).

If the bus is actively running, the host driver will

automatically wait 6 milliseconds between each

increment/decrement of the frame timing factor.

Consequently, a significant change in the frame timing factor

may take several hundred milliseconds to complete.  This is

done to ensure compliance with the USB specification

regarding how quickly a frame timing factor can be changed.

BIOS-LEVEL USB API 0.05            Page 65 of 168

Get Device Hub Information

��������������������������

Class:       Hub

Return Type: Immediate

Entry:

I14RRequestType   = 21h (I14RRTGetDvcHubInfo)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to get Info for

Return (Success):

CF = Clear

AX = 0

BL = Device Address of Hub that Device is attached to

BH = Hub Port Number that Device is attached to

CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: If the device of concern is the root hub (I14RDeviceAddress =

1), this request will return BL = 255 and BH = 255.  This is

because the root hub is not actually attached to another hub,

but is directly "attached" to the host.

Hub port numbers are 1-based (a hub never has a port number

of 0).  The port number returned will be the software port

number.  There is at least one hub manufacturer that labels

the port numbers on the hub backwards from the software port

numbers (that is software port 1 is physically labeled as

port 4, SW 2 is labeled 3, SW 3 is labeled 2, and SW 4 is

labeled 1).

Under normal circumstances, a device/interface driver would

never issue this request, since it has no need to know any

information about the hub it is attached to.

BIOS-LEVEL USB API 0.05            Page 66 of 168

Notify Host Driver of New Device Connection

�������������������������������������������

Class:       Hub

Return Type: Large

Entry:

I14RRequestType  = 24h (I14RRTNewDvcConn)

I14RHostIndex    = Host Index (0-15)

I14RHubAddress   = Device Address of Hub connection

I14RHubPort      = Port Number of Hub connection

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

I14RFlags:

I14RFlagLowSpeed (02h) = Set if Device is Low-Speed

= Clear if Device is Full-Speed

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications (Success):

AX = 0001h (OwnerCallNewDvc)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of newly attached Device (2-127)

DL = Interface Number

DH = Alternate Interface Number

Notifies device 0 owner(s) whose search criteria match alternate

interface(s) of the new device under the current configuration.

If the device is unconfigured (configuration value 0) after the

enumeration process is complete, no notifications are issued.

BIOS-LEVEL USB API 0.05            Page 67 of 168

AX = 0077h (HubCallEnumerated)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port

DL = Host Index

DH = Address of New Device (2-127, 255 = Bad Device)

Notifies the interface owner of the hub that the new device is

attached to.

Notes: This is an internal request that goes between a hub driver

and a host driver.  This request must NOT be issued by any

software other than a hub driver or a host driver.

When a hub detects a new device attachment, the hub driver

must immediately disable the newly attached device, and then

must wait AT LEAST 100 milliseconds for power to stabilize

before issuing this request to the host driver.  The host

driver will enumerate the device (reset the device, enable

it, assign it an address, configure it, enable the remote

wakeup feature (if appropriate), and gather some basic

information about the device by downloading the device and

configuration descriptors).  After the enumeration process is

complete, the host driver will issue the large return call to

the hub driver that initiated the request.  If the device

fails to respond properly during ANY part of the enumeration

process, it will be considered a "bad" device, and is

automatically disabled by the host driver.

The host driver will typically have a "reset/enumeration

queue" which allows multiple reset/enumeration requests to be

handled at the same time.

This hub notification process for an enumerated device occurs

after ALL enumerations, whether they were initiated with this

particular request, a "Reset Device Given Device Address"

request (page 104), or a "Reset Device and Change

Configuration" request (page 114).

A single device 0 owner may be notified multiple times for

the same device attachment, depending on the search criteria

of the device 0 owner, and exactly how many matching

alternate interfaces the device actually has under the

current configuration.

BIOS-LEVEL USB API 0.05            Page 68 of 168

Notify Host Driver of Device Disconnection

������������������������������������������

Class:       Hub

Return Type: Immediate

Entry:

I14RRequestType = 25h (I14RRTDvcDisc)

I14RHostIndex   = Host Index (0-15)

I14RHubAddress  = Device Address of Hub disconnection (1-127)

I14RHubPort     = Port Number of Hub disconnection

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Owner Notifications:

AX = 0002h (OwnerCallDvcDisc)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of disconnected Device (2-127)

DX = 0

Notifies interface owner(s) of all interfaces on the device.  If

the disconnected device is a hub, all device(s) downstream of

the hub are also disconnected.  Consequently, all interface

owners(s) of all downstream device(s) will also receive a

notification.

AX = 0082h (OwnerCallDvcDiscUS)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of disconnected Device (2-127)

DX = 0

Notifies interface owner(s) of all device(s) downstream of the

disconnected device.  This only applies if the disconnected

device is a hub.  Consequently, interface owner(s) of all

device(s) downstream of a disconnected hub will receive both an

OwnerCallDvcDiscUS notification (AX=0082h) and an

OwnerCallDvcDisc notification (AX=0002h).

BIOS-LEVEL USB API 0.05            Page 69 of 168

AX = 0003h (OwnerCallDvc0Disc)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of disconnected Device (2-127)

DX = 0

Notifies device 0 owner(s) whose search criteria match alternate

interface(s) of the disconnected device under the current

configuration.  If the disconnected device is a hub, all

device(s) downstream of the hub are also disconnected.

Consequently, all device 0 owners(s) whose search criteria match

alternate interface(s) of device(s) downstream of the

disconnected hub will also receive a notification.

Notes: This is an internal request that goes between a hub driver

and a host driver.  This request must NOT be issued by any

software other than a hub driver or a host driver.

The host driver AUTOMATICALLY removes all bus transactions

that are scheduled to the disconnected device(s), regardless

of end point.  Therefore, the interface owners and device 0

owners MUST NOT issue separate requests to remove the

scheduled transactions.

The root hub (device address 1) is never disconnected, so

scheduled transactions to that address will never be removed

automatically, unless the host driver is removed from memory.

When a host driver is removed from memory, a "Host

Disconnect" notification is issued to all affected owners.

A single device 0 owner or interface owner may be notified

multiple times for the same device detachment, depending on

several factors.  Those factors include the search criteria

of the device 0 owner, exactly how many matching interfaces

and alternate interfaces the device actually has under the

current configuration, and exactly how many "layers" of hubs

there are between the "owned" device and the device (hub)

that is actually disconnected.

BIOS-LEVEL USB API 0.05            Page 70 of 168

Send Hub Characteristics to Host Driver

���������������������������������������

Class:       Hub

Return Type: Immediate

Entry:

I14RRequestType = 27h (I14RRTSendHubChar)

I14RHostIndex   = Host Index (0-15)

I14RHubAddress  = Device Address of Hub

I14RHubPort     = Number of Ports on Hub (1-255)

I14RConfigValue = Hub Flags Bitmap

Bit 0 (01h) = Set if Hub supports

Per-Port-Power-Switching

Bits 1-7 = Reserved (0)

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This is an internal request that goes between a hub driver

and a host driver.  This request must NOT be issued by any

software other than a hub driver.

This request must be issued by a hub driver immediately after

it has issued the "Register As Interface Owner" request (page

121) to the host driver.

If per-port-power-switching is not supported by a particular

hub, the host driver will reject all "Power On Device Given

Device Address", "Power Off Device Given Device Address", and

"Power Reset Device Given Device Address" requests (pages 90,

92, and 94, respectively) to devices attached to that hub.

BIOS-LEVEL USB API 0.05            Page 71 of 168

Enable Device Given Hub Address and Port

����������������������������������������

Class:       Hub

Return Type: Large

Entry:

I14RRequestType  = 28h (I14RRTEnableHubPort)

I14RHostIndex    = Host Index (0-15)

I14RHubAddress   = Device Address of Hub that Port is On

I14RHubPort      = Port Number on Hub to Enable

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = 0

CX = DX = FFFFh

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = FFFFh

Owner Notifications:

AX = 0078h (HubCallEnable)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be enabled.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 72 of 168

Notes: This is an internal request that goes between a hub driver

and a host driver.  This request must NOT be issued by any

software other than a hub driver or a host driver.

The host driver temporarily passes control of the large call

over to the hub driver during the hub call.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

This request is functionally similar to an "Enable Device

Given Device Address" request (page 100), except that the

affected owner(s) are NOT notified at the end of the process.

BIOS-LEVEL USB API 0.05            Page 73 of 168

Disable Device Given Hub Address and Port

�����������������������������������������

Class:       Hub

Return Type: Large

Entry:

I14RRequestType  = 29h (I14RRTDisableHubPort)

I14RHostIndex    = Host Index (0-15)

I14RHubAddress   = Device Address of Hub that Port is On

I14RHubPort      = Port Number on Hub to Disable

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 0079h (HubCallDisable)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be disabled.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 74 of 168

Notes: This is an internal request that goes between a hub driver

and a host driver.  This request must NOT be issued by any

software other than a hub driver or a host driver.

The host driver temporarily passes control of the large call

over to the hub driver during the hub call.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

This request is functionally similar to a "Disable Device

Given Device Address" request (page 102), except that the

affected owner(s) are NOT notified at the end of the process.

BIOS-LEVEL USB API 0.05            Page 75 of 168

Reset Device Given Hub Address and Port

���������������������������������������

Class:       Hub

Return Type: Large

Entry:

I14RRequestType  = 2Ah (I14RRTResetHubPort)

I14RHostIndex    = Host Index (0-15)

I14RHubAddress   = Device Address of Hub Device is Attached to

I14RHubPort      = Port Number of Hub Device is Attached to

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 007Ah (HubCallReset)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be reset.  The hub driver must

inform the host driver when it is completed by issuing a "Large

Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 76 of 168

Notes: This is an internal request that goes between a hub driver

and a host driver.  This request must NOT be issued by any

software other than a hub driver or a host driver.

The host driver temporarily passes control of the large call

over to the hub driver during the hub call.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

The large return does not necessarily occur at the end of the

reset process, but rather may occur sometime during the

middle of the process.  The reset process may continue for

several hundred milliseconds after the large return is

issued.

This request is NOT the same thing as a "Reset Device Given

Device Address" request (page 104).  This request simply

informs the hub driver to have the hub send a reset signal on

the port of interest.  This request simply performs one "sub-

process" in an entire reset/enumeration process (which is

what the "Reset Device Given Device Address" request

performs).

BIOS-LEVEL USB API 0.05            Page 77 of 168

Suspend Device Given Hub Address and Port (Selective Suspend)

�������������������������������������������������������������

Class:       Hub

Return Type: Large

Entry:

I14RRequestType  = 2Bh (I14RRTSuspendHubPort)

I14RHostIndex    = Host Index (0-15)

I14RHubAddress   = Device Address of Hub that Port is On

I14RHubPort      = Port Number on Hub to Suspend

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 007Bh (HubCallSuspend)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be suspended.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 78 of 168

Notes: This is an internal request that goes between a hub driver

and a host driver.  This request must NOT be issued by any

software other than a hub driver or a host driver.

The host driver temporarily passes control of the large call

over to the hub driver during the hub call.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

This request is functionally similar to a "Suspend Device

Given Device Address" request (page 107), except that the

affected owner(s) are NOT notified at the end of the process.

BIOS-LEVEL USB API 0.05            Page 79 of 168

Resume Device Given Hub Address and Port (Selective Resume)

�����������������������������������������������������������

Class:       Hub

Return Type: Large

Entry:

I14RRequestType  = 2Ch (I14RRTResumeHubPort)

I14RHostIndex    = Host Index (0-15)

I14RHubAddress   = Device Address of Hub that Port is On

I14RHubPort      = Port Number on Hub to Resume

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 007Ch (HubCallResume)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be resumed.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 80 of 168

Notes: This is an internal request that goes between a hub driver

and a host driver.  This request must NOT be issued by any

software other than a hub driver or a host driver.

The host driver temporarily passes control of the large call

over to the hub driver during the hub call.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

The large return does not necessarily occur at the end of the

resume process, but rather may occur sometime during the

middle of the process.  The resume process may continue for

several hundred milliseconds after the large return is

issued.

This request is functionally similar to a "Resume Device

Given Device Address" request (page 107), except that the

affected owner(s) are NOT notified at the end of the process.

BIOS-LEVEL USB API 0.05            Page 81 of 168

Power On Device Given Hub Address and Port

������������������������������������������

Class:       Hub

Return Type: Large

Entry:

I14RRequestType  = 2Dh (I14RRTPwrOnHubPort)

I14RHostIndex    = Host Index (0-15)

I14RHubAddress   = Device Address of Hub that Port is On

I14RHubPort      = Port Number on Hub to Power On

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 007Dh (HubCallPwrOn)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be powered on.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 82 of 168

Notes: This is an internal request that goes between a hub driver

and a host driver.  This request must NOT be issued by any

software other than a hub driver or a host driver.

The host driver temporarily passes control of the large call

over to the hub driver during the hub call.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

This request is functionally similar to a "Power On Device

Given Device Address" request (page 90), except that the

affected owner(s) are NOT notified at the end of the process.

A hub driver powers on all ports while a hub is being

initialized.  Therefore, there is normally no need to issue

this request unless a "Power Off Device" request (page 92)

was previously issued.

If the hub that the port being powered on is part of does not

support per-port-power-switching, this request will be

rejected by the host driver with an error code of

I14RErrNoPortPower.

BIOS-LEVEL USB API 0.05            Page 83 of 168

Power Off Device Given Hub Address and Port

�������������������������������������������

Class:       Hub

Return Type: Large

Entry:

I14RRequestType  = 2Eh (I14RRTPwrOffHubPort)

I14RHostIndex    = Host Index (0-15)

I14RHubAddress   = Device Address of Hub that Port is On

I14RHubPort      = Port Number on Hub to Power Off

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 007Eh (HubCallPwrOff)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be powered off.  The hub

driver MUST inform the host driver when it is completed by

issuing a "Large Call Completed" request (page 154) back to the

host.

BIOS-LEVEL USB API 0.05            Page 84 of 168

Notes: This is an internal request that goes between a hub driver

and a host driver.  This request must NOT be issued by any

software other than a hub driver or a host driver.

The host driver temporarily passes control of the large call

over to the hub driver during the hub call.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

This request is functionally similar to a "Power Off Device

Given Device Address" request (page 92), except that the

affected owner(s) are NOT notified at the end of the process.

If the hub that the port being powered off is part of does

not support per-port-power-switching, this request will be

rejected by the host driver with an error code of

I14RErrNoPortPower.

If the device being powered off is bus-powered, this request

has the same functional effect as physically unplugging the

device from the bus.  If the device being powered off is

self-powered, this request should not provide any functional

effect.

BIOS-LEVEL USB API 0.05            Page 85 of 168

Power Reset Device Given Hub Address and Port

���������������������������������������������

Class:       Hub

Return Type: Large

Entry:

I14RRequestType  = 2Fh (I14RRTPwrResetHubPort)

I14RHostIndex    = Host Index (0-15)

I14RHubAddress   = Device Address of Hub that Port is On

I14RHubPort      = Port Number on Hub to Power Reset

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 007Fh (HubCallPwrReset)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be power reset.  The hub

driver MUST inform the host driver when it is completed by

issuing a "Large Call Completed" request (page 154) back to the

host.

BIOS-LEVEL USB API 0.05            Page 86 of 168

Notes: This is an internal request that goes between a hub driver

and a host driver.  This request must NOT be issued by any

software other than a hub driver or a host driver.

The host driver temporarily passes control of the large call

over to the hub driver during the hub call.  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

This request is functionally similar to a "Power Reset Device

Given Device Address" request (page 94), except that the

affected owner(s) are NOT notified at the end of the process.

If the hub that the port being power reset is part of does

not support per-port-power-switching, this request will be

rejected by the host driver with an error code of

I14RErrNoPortPower.

The hub driver has the hub hardware power off the port,

leaves it un-Powered for approximately 250 milliseconds, and

then has the hub hardware power on the port again.  If the

device being power reset is bus-powered, this request has the

same effect as physically unplugging the device from the bus,

leaving it unplugged for approximately 250 milliseconds, and

then re-inserting the device back into the hub port again.

If the device being power reset is self-powered, this request

should not provide any functional effect.

BIOS-LEVEL USB API 0.05            Page 87 of 168

Get Device Power Information

����������������������������

Class:       Power

Return Type: Immediate

Entry:

I14RRequestType   = 31h (I14RRTGetDvcPowerInfo)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to get Info for

Return (Success):

CF = Clear

AX = 0

BX = Maximum Bus Power draw of Device (milliamps)

CL = Power Flags Bitmap

Bit 0 (01h) = Set if Device is Self-Powered

Bit 6 (40h) = Set if Device is a Hub that supports

Per-Port-Power-Switching

Bit 7 (80h) = Set if Device is a Hub

Bits 1-5    = Reserved (0)

CH = Number of Ports if Device is a Hub (1-255)

= 0 if Device is not a Hub

DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: The maximum bus power draw and the self-powered status of the

device are obtained from the device's configuration

descriptor during the reset/enumeration process.  If the

device has more than one configuration, the information

returned by this request pertains only to the current

configuration.  If the device is currently unconfigured

(configuration value = 0), the information returned by this

request pertains only to the first configuration

(configuration value = 1).

There are several devices being sold where the power

information contained in the device's configuration

descriptor is incorrect (e.g., the descriptor claims that the

device is self-powered when it is actually bus-powered).

Therefore, the information returned by this request should

not necessarily be trusted for accuracy.

BIOS-LEVEL USB API 0.05            Page 88 of 168

Get Hub Power Draw

������������������

Class:       Power

Return Type: Immediate

Entry:

I14RRequestType = 32h (I14RRTGetHubPowerDraw)

I14RHostIndex   = Host Index (0-15)

I14RHubAddress  = Address of Hub to get Info for

I14RHubPort     = Hub Port number to get info for

(1-based Port Number, 0 = All Ports)

Return (Success):

CF = Clear

AX = 0

BX = Total Power drawn from Hub Ports (milliamps)

CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This indicates the total power drawn by ALL downstream

devices that claim to be bus-powered in their descriptors.

This includes devices that are "indirectly" downstream of the

hub in question, by being attached to a port on a bus-powered

hub that is downstream of the hub.

If I14RHubPort = 0, this provides the total power drawn on

all ports on the hub combined (the total power drawn by all

downstream devices).  If I14RHubPort >= 1, this provides the

total power drawn on an individual port.

There are several devices being sold where the power

information contained in the device's configuration

descriptor is incorrect (e.g., the descriptor claims that the

device is self-powered when it is actually bus-powered).

Therefore, the information returned by this request should

not necessarily be trusted for accuracy.

BIOS-LEVEL USB API 0.05            Page 89 of 168

Power On Device Given Device Address

������������������������������������

Class:       Power

Return Type: Large

Entry:

I14RRequestType   = 3Dh (I14RRTPwrOnDevice)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to Power On

I14RCallBackAddr  = Large Return Address (CS:IP)

I14RUserPktID     = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 007Dh (HubCallPwrOn)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be powered on (the port that

the device to be powered on is attached to).  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 90 of 168

AX = 000Dh (OwnerCallDvcPwrOn)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Powered On (2-127)

DX = 0

Notifies interface owner(s) of all interfaces on the device that

was powered on.  The notification will only occur if the

transaction was successful.

AX = 008Dh (OwnerCallDvcPwrOnUS)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Powered On (2-127)

DX = 0

Notifies interface owner(s) of all device(s) downstream of the

device that was powered on.  This only applies if the device

powered on is a hub.  The notification will only occur if the

transaction was successful.

Notes: This request does not actually result in anything being sent

directly to the device that is being powered on, but rather

to the hub that the device is attached to.

If the hub that the port being powered on is part of does not

support per-port-power-switching, this request will be

rejected by the host driver with an error code of

I14RErrNoPortPower.

This request is functionally similar to a "Power On Device

Given Hub Address and Port" request (page 82), except that

the affected owner(s) ARE notified at the end of the process.

A hub driver powers on all ports while a hub is being

Initialized.  Therefore, there is normally no need to issue

this request unless a "Power Off Device" request was

previously issued.

BIOS-LEVEL USB API 0.05            Page 91 of 168

Power Off Device Given Device Address

�������������������������������������

Class:       Power

Return Type: Large

Entry:

I14RRequestType   = 3Eh (I14RRTPwrOffDevice)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to Power Off

I14RCallBackAddr  = Large Return Address (CS:IP)

I14RUserPktID     = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 007Eh (HubCallPwrOff)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be powered off (the port that

the device to be powered off is attached to).  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 92 of 168

AX = 000Eh (OwnerCallDvcPwrOff)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Powered Off (2-127)

DX = 0

Notifies interface owner(s) of all interfaces on the device that

was powered off.  The notification will only occur if the

transaction was successful.

AX = 008Eh (OwnerCallDvcPwrOffUS)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Powered Off (2-127)

DX = 0

Notifies interface owner(s) of all device(s) downstream of the

device that was powered off.  This only applies if the device

powered off is a hub.  The notification will only occur if the

transaction was successful.

Notes: This request does not actually result in anything being sent

directly to the device that is being powered off, but rather

to the hub that the device is attached to.

If the hub that the port being powered off is part of does

not support per-port-power-switching, this request will be

rejected by the host driver with an error code of

I14RErrNoPortPower.

This request is functionally similar to a "Power Off Device

Given Hub Address and Port" request (page 84), except that

the affected owner(s) ARE notified at the end of the process.

If the device being powered off is bus-powered, this request

has the same functional effect as physically unplugging the

device from the bus.  If the device being powered off is

self-powered, this request will not provide any functional

effect.

BIOS-LEVEL USB API 0.05            Page 93 of 168

Power Reset Device Given Device Address

���������������������������������������

Class:       Power

Return Type: Large

Entry:

I14RRequestType   = 3Fh (I14RRTPwrResetDevice)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to Power Reset

I14RCallBackAddr  = Large Return Address (CS:IP)

I14RUserPktID     = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 007Fh (HubCallPwrReset)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be power reset (the port that

the device to be power reset is attached to).  The hub driver

MUST inform the host driver when it is completed by issuing a

"Large Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 94 of 168

AX = 000Fh (OwnerCallDvcPwrReset)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Power Reset (2-127)

DX = 0

Notifies interface owner(s) of all interfaces on the device that

was power reset.  The notification will only occur if the

transaction was successful.

AX = 008Fh (OwnerCallDvcPwrResetUS)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Power Reset (2-127)

DX = 0

Notifies interface owner(s) of all device(s) downstream of the

device that was power reset.  This only applies if the device

power reset is a hub.  The notification will only occur if the

transaction was successful.

Notes: This request does not actually result in anything being sent

directly to the device that is being power reset, but rather

to the hub that the device is attached to.

If the hub that the port being power reset is part of does

not support per-port-power-switching, this request will be

rejected by the host driver with an error code of

I14RErrNoPortPower.

This request is functionally similar to a "Power Reset Device

Given Hub Address and Port" request (page 86), except that

the affected owner(s) ARE notified at the end of the process.

The hub driver has the hub hardware power off the port,

leaves it un-Powered for approximately 250 milliseconds, and

then has the hub hardware power on the port again.  If the

device being power reset is bus-powered, this request has the

same effect as physically unplugging the device from the bus,

leaving it unplugged for approximately 250 milliseconds, and

then re-inserting the device back into the hub port again.

If the device being power reset is self-powered, this request

will not provide any functional effect.

BIOS-LEVEL USB API 0.05            Page 95 of 168

Get Device Class Information

����������������������������

Class:       Device

Return Type: Immediate

Entry:

I14RRequestType   = 41h (I14RRTGetDvcClassInfo)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to get Info for

Return (Success):

CF = Clear

AX = 0

BL = Device Class

BH = Device SubClass

CL = Device Protocol

CH = USB Version (BCD: High Nibble = Major, Low Nibble = Minor)

DL = Speed Flags Bitmap

Bit 0 (01h) = Set if Device is Low-Speed

Bits 1-7    = Reserved (0)

DH = FFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: The class, subclass, protocol, and USB version are obtained

from the device descriptor during the reset/enumeration

process.

The USB version is contained in the device descriptor as a

word, which is "compressed" into a single byte before being

returned by this request.  For example, version 01.10 (0110h

in the device descriptor) is "compressed" (the high and low

nibbles are discarded) and returned as 11h (version 1.1).

This format (a byte as opposed to a word) is consistent with

the USB version as returned by the host hardware on the PCI

bus.

BIOS-LEVEL USB API 0.05            Page 96 of 168

Get Device Vendor Information

�����������������������������

Class:       Device

Return Type: Immediate

Entry:

I14RRequestType   = 42h (I14RRTGetDvcVendInfo)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to get Info for

Return (Success):

CF = Clear

AX = 0

BX = Device Vendor ID

CX = Device Product ID

DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: The vendor ID and product ID are obtained from the device

descriptor during the reset/enumeration process.

For the root hub (address 1), the vendor ID and product ID

are associated with the PCI host controller (a PCI device,

not a USB device).  Therefore, the vendor ID and product ID

will be PCI ID's, not USB ID's (it is not possible to

"translate" a PCI ID into a USB ID).  This is the same data

returned by a "Get Host Vendor Information" request (page

46).

BIOS-LEVEL USB API 0.05            Page 97 of 168

Get Device Status Information

�����������������������������

Class:       Device

Return Type: Immediate

Entry:

I14RRequestType   = 43h (I14RRTGetDvcStatus)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to get Info for

Return (Success):

CF = Clear

AX = 0

BL = Current Configuration Value for Device

BH = Status Flags Bitmap

Bit 0 (01h) = Set if Device is "Bad" (a problem occurred

during the Reset/Enumeration Process)

Bit 1 (02h) = Set if Device supports Remote Wakeup Feature

Bit 2 (04h) = Set if Remote Wakeup Feature is Enabled

Bit 3 (08h) = Set if Test Mode is Enabled

Bits 4-6    = Reserved (0)

Bit 7 (80h) = Set if Device is currently in the middle of the

Reset/Enumeration process

CL = Last Completed Stage of Reset/Enumeration Process

CH = FFh

DX = Error Code Associated with Last Completed Stage of

Reset/Enumeration Process

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: The host driver must monitor the appropriate control requests

in "real time" so that it can process this request as an

immediate return.  The host must NOT send a control packet

across the bus requesting status(es) directly from the device

before processing this request.

The remote wakeup feature support status (bit 1 of the BH

register) is obtained from the device's configuration

descriptor during the reset/enumeration process.  If the

device has more than one configuration, the information

returned by this request pertains only to the current

configuration.  If the device is currently unconfigured

(configuration value = 0), the information returned by this

request pertains only to the first configuration

(configuration value = 1).

BIOS-LEVEL USB API 0.05            Page 98 of 168

The remote wakeup feature enabled status (bit 2 of the BH

register) is obtained by monitoring the device's response to

the most recent "Set Feature: Remote Wakeup" or "Clear

Feature: Remote Wakeup" control request.  It does not

necessarily indicate the true enabled status of the remote

wakeup feature.  A device which does not support the remote

wakeup feature (as indicated in bit 1 of the BH register)

should reject any request associated with the feature, but

many devices do not.  The true status of the remote wakeup

feature is indicated by the combination of bits 1 and 2.

The test mode enabled status (bit 3 of the BH register) is

obtained by monitoring the device's response to the most

recent "Set Feature: Test Mode" or "Clear Feature: Test Mode"

control request.  It may or may not indicate the true test

mode enabled status, given that many devices respond

affirmatively to feature control requests that they do not

support and should reject.

If bit 7 (80h) of the value returned in BH is set, the device

is currently in the middle of the enumeration process.  Any

data associated with the device returned by this, or any

other, request may be incomplete or erroneous until the

enumeration process has been completed in its entirety.

If bit 0 (01h) of the value returned in BH is set (indicating

that the device is "bad"), the values returned in CL and DX

can be used as a troubleshooting tool to help determine where

and how the device failed during the enumeration process.

The precise meaning of the values returned in CL & DX are

specific to the host driver software used.  They are of

little value other than highly technical troubleshooting.

Refer to the specific host driver software documentation for

details.

BIOS-LEVEL USB API 0.05            Page 99 of 168

Enable Device Given Device Address

����������������������������������

Class:       Device

Return Type: Large

Entry:

I14RRequestType   = 48h (I14RRTEnableDevice)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to Enable

I14RCallBackAddr  = Large Return Address (CS:IP)

I14RUserPktID     = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 0078h (HubCallEnable)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be enabled (the port that the

device to be enabled is attached to).  The hub driver MUST

inform the host driver when it is completed by issuing a "Large

Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 100 of 168

AX = 0008h (OwnerCallDvcEnable)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Enabled (2-127)

DX = 0

Notifies interface owner(s) of all interfaces on the device that

was enabled.  The notification will only occur if the

transaction was successful.

AX = 0088h (OwnerCallDvcEnableUS)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Enabled (2-127)

DX = 0

Notifies interface owner(s) of all device(s) downstream of the

device that was enabled.  This only applies if the enabled

device is a hub.  The notification will only occur if the

transaction was successful.

Notes: This request does not actually result in anything being sent

directly to the device that is being enabled, but rather to

the hub that the device is attached to.

This request is functionally similar to an "Enable Device

Given Hub Address and Port" request (page 72), except that

the affected owner(s) ARE notified at the end of the process.

If a device is disabled, enabling it will NOT cause it

"recover" and start running normally again.  A device must be

reset to "recover" from being disabled.

BIOS-LEVEL USB API 0.05            Page 101 of 168

Disable Device Given Device Address

�����������������������������������

Class:       Device

Return Type: Large

Entry:

I14RRequestType   = 49h (I14RRTDisableDevice)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to Disable

I14RCallBackAddr  = Large Return Address (CS:IP)

I14RUserPktID     = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 0079h (HubCallDisable)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be disabled (the port that the

device to be disabled is attached to).  The hub driver MUST

inform the host driver when it is completed by issuing a "Large

Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 102 of 168

AX = 0009h (OwnerCallDvcDisable)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Disabled (2-127)

DX = 0

Notifies interface owner(s) of all interfaces on the device that

was disabled.  The notification will only occur if the

transaction was successful.

AX = 0089h (OwnerCallDvcDisableUS)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Disabled (2-127)

DX = 0

Notifies interface owner(s) of all device(s) downstream of the

device that was disabled.  This only applies if the disabled

device is a hub.  The notification will only occur if the

transaction was successful.

Notes: This request does not actually result in anything being sent

directly to the device that is being disabled, but rather to

the hub that the device is attached to.

This request is functionally similar to an "Disable Device

Given Hub Address and Port" request (page 74), except that

the affected owner(s) ARE notified at the end of the process.

If a device is disabled, enabling it will NOT cause it

"recover" and start running normally again.  A device must be

reset to "recover" from being disabled.

BIOS-LEVEL USB API 0.05            Page 103 of 168

Reset Device Given Device Address

���������������������������������

Class:       Device

Return Type: Large

Entry:

I14RRequestType   = 4Ah (I14RRTResetDevice)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to Reset

I14RCallBackAddr  = Large Return Address (CS:IP)

I14RUserPktID     = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 000Ah (OwnerCallDvcReset)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device being Reset (2-127)

DX = 0

Notifies interface owner(s) of all interfaces on the device that

is being reset.

BIOS-LEVEL USB API 0.05            Page 104 of 168

AX = 008Ah (OwnerCallDvcResetUS)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device being Reset (2-127)

DX = 0

Notifies interface owner(s) of all device(s) downstream of the

device being reset.  This only applies if the device being reset

is a hub.

AX = 0002h (OwnerCallDvcDisc)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device being Reset (2-127)

DX = 0

Notifies interface owner(s) of all interfaces on the device.  If

the device being reset is a hub, all device(s) downstream of the

hub are also disconnected.  Consequently, all interface

owners(s) of all downstream device(s) will also receive a

notification.

AX = 0082h (OwnerCallDvcDiscUS)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device being Reset (2-127)

DX = 0

Notifies interface owner(s) of all device(s) downstream of the

device being reset.  This only applies if the device being reset

is a hub.  Consequently, interface owner(s) of all device(s)

downstream of a hub being reset will receive both an

OwnerCallDvcDiscUS notification (AX=0082h) and an

OwnerCallDvcDisc notification (AX=0002h).

AX = 0003h (OwnerCallDvc0Disc)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device being Reset (2-127)

DX = 0

Notifies device 0 owner(s) whose search criteria match alternate

interface(s) of the device being reset under the current

configuration.  If the device being reset is a hub, all

device(s) downstream of the hub are also disconnected.

Consequently, all device 0 owners(s) whose search criteria match

alternate interface(s) of device(s) downstream of the hub being

reset will also receive a notification.

BIOS-LEVEL USB API 0.05            Page 105 of 168

Notes: This request initiates an entire reset/enumeration process on

a device.  This process involves several different "sub-

processes", resulting in several different notifications

being sent to the owner of the hub that the device being

reset is attached to.  Those notifications can include

HubCallDisable (0079h), HubCallReset (007Ah), HubCallEnable

(0078h), and HubCallEnumerated (0077h).

This request is effectively a "software version" of

momentarily detaching the device from the hub and then

reinserting it again.  The same process is executed that

occurs when a device is first attached to the bus.

The device remains continuously attached to bus power during

the entire reset/enumeration process.

After the reset/enumeration process is complete, the device

may not necessarily have the same address it did before the

request was initiated.

After the reset/enumeration process is complete, the device

will normally be set to the "default" configuration

(configuration value = 1).  If the device needs to be

configured to any other value, including configuration value

= 0 (unconfigured), you must use the "Reset Device and Set

New Configuration" request (page 114).

The large return does not necessarily occur at the end of the

reset/enumeration process, but rather may occur sometime

during the middle of the process.  The reset/enumeration

process may continue for several hundred milliseconds after

the large return is issued.

This request is not the same thing as a "Reset Device Given

Hub Address and Port" request (page 76).  This request

initiates an entire reset/enumeration process on a device,

while the "Reset Device Given Hub Address and Port" request

simply performs one of the "sub-processes" involved in the

entire reset/enumeration process.

If a device is disabled, enabling it will NOT cause it

"recover" and start running normally again.  A device must be

reset to "recover" from being disabled.

BIOS-LEVEL USB API 0.05            Page 106 of 168

Suspend Device Given Device Address (Selective Suspend)

�������������������������������������������������������

Class:       Device

Return Type: Large

Entry:

I14RRequestType   = 4Bh (I14RRTSuspendDevice)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to Suspend

I14RCallBackAddr  = Large Return Address (CS:IP)

I14RUserPktID     = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 007Bh (HubCallSuspend)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be suspended (the port that

the device to be suspended is attached to).  The hub driver MUST

inform the host driver when it is completed by issuing a "Large

Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 107 of 168

AX = 000Bh (OwnerCallDvcSuspend)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Suspended (2-127)

DX = 0

Notifies interface owner(s) of all interfaces on the device that

was suspended.  The notification will only occur if the

transaction was successful.

AX = 008Bh (OwnerCallDvcSuspendUS)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Suspended (2-127)

DX = 0

Notifies interface owner(s) of all device(s) downstream of the

device that was suspended.  This only applies if the suspended

device is a hub.  The notification will only occur if the

transaction was successful.

Notes: This request does not actually result in anything being sent

directly to the device that is being suspended, but rather to

the hub that the device is attached to.

This request is functionally similar to an "Suspend Device

Given Hub Address and Port (Selective Suspend)" request (page

80), except that the affected owner(s) ARE notified at the

end of the process.

BIOS-LEVEL USB API 0.05            Page 108 of 168

Resume Device Given Device Address (Selective Resume)

�����������������������������������������������������

Class:       Device

Return Type: Large

Entry:

I14RRequestType   = 4Ch (I14RRTResumeDevice)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to Resume

I14RCallBackAddr  = Large Return Address (CS:IP)

I14RUserPktID     = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 007Ch (HubCallResume)

BX = I14RUserPktID of Hub Owner

CL = Hub Address

CH = Hub Port Number

DL = Large Call Index

DH = Host Index (0-15)

Notifies hub owner of the port to be resumed (the port that the

device to be resumed is attached to).  The hub driver MUST

inform the host driver when it is completed by issuing a "Large

Call Completed" request (page 154) back to the host.

BIOS-LEVEL USB API 0.05            Page 109 of 168

AX = 000Ch (OwnerCallDvcResume)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Resumed (2-127)

DX = 0

Notifies interface owner(s) of all interfaces on the device that

was resumed.  The notification will only occur if the

transaction was successful.

AX = 008Bh (OwnerCallDvcResumeUS)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device that was Resumed (2-127)

DX = 0

Notifies interface owner(s) of all device(s) downstream of the

device that was resumed.  This only applies if the resumed

device is a hub.  The notification will only occur if the

transaction was successful.

Notes: This request does not actually result in anything being sent

directly to the device that is being resumed, but rather to

the hub that the device is attached to.

The large return does not necessarily occur at the end of the

resume, but rather may occur sometime during the middle of

the process.  The resume process may continue for several

hundred milliseconds after the large return is issued.

This request is functionally similar to an "Resume Device

Given Hub Address and Port (Selective Resume)" request (page

80), except that the affected owner(s) ARE notified at the

end of the process.

BIOS-LEVEL USB API 0.05            Page 110 of 168

Configuring Interface

���������������������

Class:       Configuration

Return Type: Immediate

Entry:

I14RRequestType   = 51h (I14RRTConfigingIntf)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device being configured (2-127)

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This request is issued by a device driver while it is

"configuring" an interface (downloading descriptors,

selecting alternate interfaces, etc.).  The purpose of the

request is to "warn" other device drivers that might be

attempting to perform similar functions on other interfaces

of the same device that they should wait, since many devices

become "confused" if multiple configuration processes are

occurring simultaneously.

Technically, this request does not need to be issued while

configuring an interface on a device that has only one

interface.  However, it is highly recommended in all

situations.

If another "Configuring Interface" request is received for

any interface on the same device while another "Configuring

Interface" request is in process, the second request will

return with an error code of I14RErrIntfInConfig.

When the configuration process is complete, the Requester

should issue a "Configuring Interface Complete" request (page

113).  If a "Configuring Interface Complete" request is not

received by the host driver within 5 seconds after receiving

the "Configuring Interface" request, the host driver will

automatically terminate the configuration process and

essentially issue its own "Configuring Interface Complete"

request.

BIOS-LEVEL USB API 0.05            Page 111 of 168

The process performed by this request is similar in concept

to ownership registration, but is temporary rather than

permanent, and affects the entire device instead of only a

single interface.

BIOS-LEVEL USB API 0.05            Page 112 of 168

Configuring Interface Complete

������������������������������

Class:       Configuration

Return Type: Immediate

Entry:

I14RRequestType   = 52h (I14RRTConfigingIntfDone)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device done configuring (2-127)

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This request is issued to terminate a "Configuring Interface"

request (page 111).  If a "Configuring Interface Complete"

request is not received by the host driver within 5 seconds

after receiving the "Configuring Interface" request, the host

driver will automatically terminate the configuration process

and essentially issue its own "Configuring Interface

Complete" request.

BIOS-LEVEL USB API 0.05            Page 113 of 168

Reset Device and Set New Configuration

��������������������������������������

Class:       Configuration

Return Type: Large

Entry:

I14RRequestType   = 58h (I14RRTSetNewConfig)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = Address of Device to Reconfigure

I14RConfigValue   = New Configuration Value to set (0-255)

I14RCallBackAddr  = Large Return Address (CS:IP)

I14RUserPktID     = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

BX = I14RUserPktID of Requester

AX = CX = DX = 0

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications:

AX = 000Ah (OwnerCallDvcReset)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device being Reconfigured (2-127)

DX = 0

Notifies interface owner(s) of all interfaces on the device that

is being reconfigured.

BIOS-LEVEL USB API 0.05            Page 114 of 168

AX = 008Ah (OwnerCallDvcResetUS)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device being Reconfigured (2-127)

DX = 0

Notifies interface owner(s) of all device(s) downstream of the

device being reconfigured.  This only applies if the device

being reconfigured is a hub.

AX = 0002h (OwnerCallDvcDisc)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device being Reconfigured (2-127)

DX = 0

Notifies interface owner(s) of all interfaces on the device.  If

the device being reconfigured is a hub, all device(s) downstream

of the hub are also disconnected.  Consequently, all interface

owners(s) of all downstream device(s) will also receive a

notification.

AX = 0082h (OwnerCallDvcDiscUS)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device being Reconfigured (2-127)

DX = 0

Notifies interface owner(s) of all device(s) downstream of the

device being reconfigured.  This only applies if the device

being reconfigured is a hub.  Consequently, interface owner(s)

of all device(s) downstream of a hub being reconfigured will

receive both an OwnerCallDvcDiscUS notification (AX=0082h) and

an OwnerCallDvcDisc notification (AX=0002h).

AX = 0003h (OwnerCallDvc0Disc)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = Device Address of Device being Reconfigured (2-127)

DX = 0

Notifies device 0 owner(s) whose search criteria match alternate

interface(s) of the device being reconfigured under the current

(old) configuration.  If the device being reconfigured is a hub,

all device(s) downstream of the hub are also disconnected.

Consequently, all device 0 owners(s) whose search criteria match

alternate interface(s) of device(s) downstream of the hub being

reconfigured will also receive a notification.

BIOS-LEVEL USB API 0.05            Page 115 of 168

Notes: This request is the ONLY way to change the configuration

value of a device, or to set the configuration value of a

device to anything other than the default value of 1.

This request is functionally the same as a "Reset Device

Given Device Address" request (page 104), except for the

portion of the reset/enumeration process where the device is

configured.  For this request, the host driver will attempt

to set the configuration to the value in I14RConfigValue when

the request was issued.  If the device does not accept the

requested configuration value for any reason, the host driver

will then attempt to set the default configuration

(configuration value = 1).

This request initiates an entire reset/enumeration process on

a device.  This process involves several different "sub-

processes", resulting in several different notifications

being sent to the owner of the hub that the device being

reset is attached to.  Those notifications can include

HubCallDisable (0079h), HubCallReset (007Ah), HubCallEnable

(0078h), and HubCallEnumerated (0077h).

This request is effectively a "software version" of

momentarily detaching the device from the hub and then

reinserting it again.  The same process is executed that

occurs when a device is first attached to the bus.

After the reset/enumeration process is complete, the device

may not necessarily have the same address it did before the

request was initiated.

The device remains continuously attached to bus power during

the entire reset/enumeration process.

BIOS-LEVEL USB API 0.05            Page 116 of 168

Find Registered Interface

�������������������������

Class:       Interface

Return Type: Immediate

Entry:

I14RRequestType  = 62h (I14RRTFindRegIntf)

I14RHostIndex    = Host Index (0-15, 255 = All Hosts)

I14RSearchIndex  = Search Index to use (0-255)

I14RVendorID     = Vendor ID of Dvc (FFFFh or 0000h = Don't Care)

I14RProductID    = Product ID of Dvc (FFFFh or 0000h = Don't Care)

I14RDvcClass     = Device Class of Device (FFh = Don't Care)

I14RDvcSubClass  = Device SubClass of Device (FFh = Don't Care)

I14RDvcProtocol  = Device Protocol of Device (FFh = Don't Care)

I14RIntfClass    = Interface Class of Alternate Interface

(FFh = Don't Care)

I14RIntfSubClass = Interface SubClass of Alternate Interface

(FFh = Don't Care)

I14RIntfProtocol = Interface Protocol of Alternate Interface

(FFh = Don't Care)

Return (Success):

CF = Clear

AX = 0

BX = FFFFh

CL = Host Index that Device is attached to (0-15)

CH = Device Address (2-127)

DL = Interface Number (0-255)

DH = Alternate Interface Number (0-255)

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: Search index 0 will find the "first" registered interface

matching the specified criteria.  The "first" interface is

the one with the lowest alternate interface number,

associated with the lowest interface number, on the device

with the lowest address number, attached to the host with the

lowest index number.  Search index 1 will find the "second"

matching registered interface, etc.

BIOS-LEVEL USB API 0.05            Page 117 of 168

If one (or more) of the search criteria is "Don't Care"

(FFFFh or 0000h in the vendor/product ID, or FFh in the

device/interface class/subclass/protocol), it will ALWAYS

"match" the corresponding field from the actual device.  It

is not possible to specifically look for an FFFFh or 0000h in

the actual vendor/product ID fields, nor is it possible to

specifically look for an FFh (vendor-specific) value in the

actual device/interface class/subclass/protocol fields.

This request finds all matching alternate interfaces, even if

those alternate interfaces are not currently selected.

It is possible to look for an interface on a specific host,

though I14RHostIndex for this request would normally be set

to 255 (all hosts).

This request finds interface(s) that already have a

registered owner.  The only type of software that should need

to look for already-registered interfaces are vendor-specific

or product-specific drivers, which are wanting to "wrest"

control of an interface from a more general-purpose device

driver.  This request would not typically be used by "normal"

device drivers.

To process this request effectively with the possibility of

multiple, semi-independent host drivers, each host driver

will modify the ORIGINAL I14RSearchIndex value as it finds

matching interface(s).  If I14RSearchIndex is not 0 when a

host driver finds a matching interface, it will simply

decrement I14RSearchIndex in the ORIGINAL request data

structure that it received, and respond as if it did NOT find

a matching Interface.  If the original request was issued

with I14RHostIndex = 255 (all hosts), the host driver that

received the original request will continue to "pass on" the

request to each host driver individually, using the

I14RSearchIndex as it was returned (and possibly modified) by

the previous host driver.  Only when a matching interface is

found and I14RSearchIndex is 0 will a host driver return a

successful result.

BIOS-LEVEL USB API 0.05            Page 118 of 168

Find Unregistered Interface

���������������������������

Class:       Interface

Return Type: Immediate

Entry:

I14RRequestType  = 63h (I14RRTFindUnRegIntf)

I14RHostIndex    = Host Index (0-15, 255 = All Hosts)

I14RSearchIndex  = Search Index to use (0-255)

I14RVendorID     = Vendor ID of Dvc (FFFFh or 0000h = Don't Care)

I14RProductID    = Product ID of Dvc (FFFFh or 0000h = Don't Care)

I14RDvcClass     = Device Class of Device (FFh = Don't Care)

I14RDvcSubClass  = Device SubClass of Device (FFh = Don't Care)

I14RDvcProtocol  = Device Protocol of Device (FFh = Don't Care)

I14RIntfClass    = Interface Class of Alternate Interface

(FFh = Don't Care)

I14RIntfSubClass = Interface SubClass of Alternate Interface

(FFh = Don't Care)

I14RIntfProtocol = Interface Protocol of Alternate Interface

(FFh = Don't Care)

Return (Success):

CF = Clear

AX = 0

BX = FFFFh

CL = Host Index that Device is attached to (0-15)

CH = Device Address (2-127)

DL = Interface Number (0-255)

DH = Alternate Interface Number (0-255)

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: Search index 0 will find the "first" unregistered interface

matching the specified criteria.  The "first" interface is

the one with the lowest alternate interface number,

associated with the lowest interface number, on the device

with the lowest address number, attached to the host with the

lowest index number.  Search index 1 will find the "second"

matching unregistered interface, etc.

BIOS-LEVEL USB API 0.05            Page 119 of 168

If one (or more) of the search criteria is "Don't Care"

(FFFFh or 0000h in the vendor/product ID, or FFh in the

device/interface class/subclass/protocol), it will ALWAYS

"match" the corresponding field from the actual device.  It

is not possible to specifically look for an FFFFh or 0000h in

the actual vendor/product ID fields, nor is it possible to

specifically look for an FFh (vendor-specific) value in the

actual device/interface class/subclass/protocol fields.

This request finds all matching alternate interfaces, even if

those alternate interfaces are not currently selected.

It is possible to look for an interface on a specific host,

though I14RHostIndex for this request would normally be set

to 255 (all hosts).

To process this request effectively with the possibility of

multiple, semi-independent host drivers, each host driver

will modify the ORIGINAL I14RSearchIndex value as it finds

matching interface(s).  If I14RSearchIndex is not 0 when a

host driver finds a matching interface, it will simply

decrement I14RSearchIndex in the ORIGINAL request data

structure that it received, and respond as if it did NOT find

a matching interface.  If the original request was issued

with I14RHostIndex = 255 (all hosts), the host driver that

received the original request will continue to "pass on" the

request to each host driver individually, using the

I14RSearchIndex as it was returned (and possibly modified) by

the previous host driver.  Only when a matching interface is

found and I14RSearchIndex is 0 will a host driver return a

successful result.

BIOS-LEVEL USB API 0.05            Page 120 of 168

Register As Interface Owner

���������������������������

Class:       Interface

Return Type: Immediate

Entry:

I14RRequestType   = 64h (I14RRTRegIntfOwner)

I14RHostIndex     = Host Index Device is attached to (0-15)

I14RDeviceAddress = Address of Device Interface is on (2-127)

I14RInterfaceNum  = Interface Number to Register

I14RCallBackAddr  = Notification Call Back Address (CS:IP)

(Interface Owner)

I14RUserPktID     = Value to return in BX on Notifications

Return (Success):

CF = Clear

AX = 0

BX = DX = FFFFh

CX = Interface Owner Handle

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Owner Notifications:

AX = (see NOTIFICATION CODES Table on page 10)

BX = I14RUserPktID of Interface Owner

CX = Depends on Notification Code

DX = Depends on Notification Code

The interface owner is notified when changes occur somewhere on

the bus that would be of interest to the interface owner.  For

example, an interface owner is notified if the device that the

interface is on (or an upstream device) is disabled, enabled,

reset, suspended, resumed, or detached from the bus.  Most any

notification detailed in the NOTIFICATION CODES table (page 10)

can be issued to an interface owner.

Notes: There can be only one registered owner of an interface, no

matter how many alternate interfaces are associated with that

particular interface.

The interface owner handle (the value returned in CX) must be

"returned" (in I14RRequestHandle) in the "Unregister As

Interface Owner" request (page 126).

The requester will remain the interface owner until the host

driver receives an "Unregister As Interface Owner" request

(page 126), or until the interface (device) is disconnected.

BIOS-LEVEL USB API 0.05            Page 121 of 168

No software other than the registered owner of an interface

can issue requests that will directly affect the operation of

an interface.  For instance, a non-owner could issue a

request to download a device or interface descriptor from the

device, but could NOT issue a request to select a different

alternate interface.

If the interface (device) is a hub, the hub-specific

notifications in the NOTIFICATION CODE TABLE (page 10) are

issued to the registered interface owner.  There is not a

special "Hub Owner Registration" process.

The value in the I14RRequestType field for this request

(I14RRTRegIntfOwner = 64h) is the same value used in a

"Register as Device 0 (New Interface) Owner" request (page

123).  In this request, I14RHostIndex must be a legitimate

host index (0-15), and I14RDeviceAddress must be a legitimate

device address (2-127).

BIOS-LEVEL USB API 0.05            Page 122 of 168

Register As Device 0 (New Interface) Owner

������������������������������������������

Class:       Interface

Return Type: Immediate

Entry:

I14RRequestType   = 64h (I14RRTRegIntfOwner)

I14RHostIndex     = 255 (All Hosts)

I14RDeviceAddress = 0 (Newly Attached Devices)

I14RVendorID      = Dvc Vendor ID (FFFFh or 0000h = Don't Care)

I14RProductID     = Dvc Product ID (FFFFh or 0000h = Don't Care)

I14RDvcClass      = Device Class of Device (FFh = Don't Care)

I14RDvcSubClass   = Device SubClass of Device (FFh = Don't Care)

I14RDvcProtocol   = Device Protocol of Device (FFh = Don't Care)

I14RIntfClass     = Interface Class of Alternate Interface

(FFh = Don't Care)

I14RIntfSubClass  = Interface SubClass of Alternate Interface

(FFh = Don't Care)

I14RIntfProtocol  = Interface Protocol of Alternate Interface

(FFh = Don't Care)

I14RCallBackAddr  = Notification Call Back Address (CS:IP)

(Device 0 Owner)

I14RUserPktID     = Value to return in BX on Notifications

Return (Success):

CF = Clear

AX = 0

BX = DX = FFFFh

CX = Device 0 Owner Handle (0-127)

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Owner Notifications:

AX = 0001h (OwnerCallNewDvc)

BX = I14RUserPktID of Device 0 Owner

CL = Host Index (0-15)

CH = Device Address of Newly Attached Device (2-127)

DL = Interface Number that matching Alternate Interface is on

DH = Matching Alternate Interface Number

Notifies all registered device 0 owners whose search criteria

match the characteristics of the newly attached device

(I14RVendorID, I14RProductID, I14RDvcClass, I14RDvcSubClass,

I14RDvcProtocol, I14RIntfClass, I14RIntfSubClass, and

I14RIntfProtocol).

BIOS-LEVEL USB API 0.05            Page 123 of 168

AX = 0003h (OwnerCallDvc0Disc)

BX = I14RUserPktID of Device 0 Owner

CL = Host Index (0-15)

CH = Device Address of Detached Device (2-127)

DL = Interface Number that matching Alternate Interface is on

DH = Matching Alternate Interface Number

Notifies all registered device 0 owners whose search criteria

match the characteristics of the detached device (I14RVendorID,

I14RProductID, I14RDvcClass, I14RDvcSubClass, I14RDvcProtocol,

I14RIntfClass, I14RIntfSubClass, and I14RIntfProtocol).

Notes: This request registers an "outside" program to be notified

when an interface (device) is first attached to or detached

from the bus.  Software can (and should) periodically poll

the host driver(s) to look for new devices, but this request

provides a means of immediate notification.

Depending on the total number of alternate interfaces a

device has, and exactly how the search criteria are defined,

a device 0 owner may be notified multiple times for a single

device attachment/detachment.

The device 0 owner handle (the value returned in CX) must be

"returned" (in I14RRequestHandle) in the "Unregister As

Device 0 Owner" request (page 127).

The requester will remain registered as a device 0 owner

until the host driver receives an "Unregister As Device 0

Owner" request (page 127).

If one (or more) of the search criteria is "Don't Care"

(FFFFh or 0000h in the vendor/product ID, or FFh in the

device/interface class/subclass/protocol), it will ALWAYS

"match" the corresponding field from the actual device.  It

is not possible to specifically look for a 0000h or FFFFh in

the actual vendor/product ID fields, nor is it possible to

specifically look for an FFh (vendor-specific) value in the

actual device/interface class/subclass/protocol fields.

This request will match all alternate interfaces on a device,

even if those alternate interfaces are not currently

selected.

The value in the I14RRequestType field for this request

(I14RRTRegIntfOwner = 64h) is the same value used in a

"Register as Interface Owner" request (page 121).  In this

request, I14RHostIndex must be 255 (all hosts), and

I14RDeviceAddress must be 0.

BIOS-LEVEL USB API 0.05            Page 124 of 168

The first host driver to receive this request will assign the

device 0 owner handle (the 0-based index number in the device

0 owner table).  It will then place the assigned handle

number in I14RRequestHandle, "translate" the original request

into an "InterHost Register As Device 0 Owner" request (page

159), and issue the inter-host request to all host drivers

individually.

BIOS-LEVEL USB API 0.05            Page 125 of 168

Unregister As Interface Owner

�����������������������������

Class:       Interface

Return Type: Immediate

Entry:

I14RRequestType   = 65h (I14RRTUnRegIntfOwner)

I14RHostIndex     = Host Index Device is attached to (0-15)

I14RRequestHandle = Interface Owner Handle

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: The interface owner handle is the value that was returned in

the CX register during the "Register as Interface Owner"

request (page 121).

The value in the I14RRequestType field for this request

(I14RRTUnRegIntfOwner = 65h) is the same value used in a

"UnRegister as Device 0 (New Interface) Owner" request (page

127).  In this request, I14RHostIndex must be a legitimate

host index (0-15).

BIOS-LEVEL USB API 0.05            Page 126 of 168

Unregister As Device 0 (New Interface) Owner

��������������������������������������������

Class:       Interface

Return Type: Immediate

Entry:

I14RRequestType   = 65h (I14RRTUnRegIntfOwner)

I14RHostIndex     = 255 (All Hosts)

I14RRequestHandle = Device 0 Owner Handle (0-127)

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: The device 0 owner handle is the value that was returned in

the CX register during the "Register as Device 0 (New

Interface) Owner" request (page 123).

The value in the I14RRequestType field for this request

(I14RRTUnRegIntfOwner = 65h) is the same value used in a

"UnRegister as Interface Owner" request (page 126).  In this

request, I14RHostIndex must be 255 (all hosts).

The first host driver to receive this request will

"translate" the original request into an "InterHost

UnRegister As Device 0 Owner" request (page 162), and issue

the inter-host request to all host drivers individually.

BIOS-LEVEL USB API 0.05            Page 127 of 168

Unregister Current Interface Owner and Don't Look

�������������������������������������������������

Class:       Interface

Return Type: Immediate

Entry:

I14RRequestType   = 68h (I14RRTIntfDontLook)

I14RHostIndex     = Host Index Device is attached to (0-15)

I14RDeviceAddress = Address of Device Interface is on (2-127)

I14RInterfaceNum  = Interface Number to UnRegister

I14RTimeout       = Don't Look Timeout value to use (deciseconds)

(0-255): Only the Least Significant Byte of

I14RTimeout is used.

The Most Significant Byte of the

Word is ignored.

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Owner Notifications:

AX = 0007h (OwnerCallDontLook)

BX = I14RUserPktID of Interface Owner

CL = Host Index (0-15)

CH = Address of Device that Interface is On (2-127)

DL = Interface Number to UnRegister

DH = Don't Look Timeout value to use (deciseconds) (1-255)

Notifies the existing interface owner that it needs to

relinquish control (unregister as owner) of the interface, and

to not try to regain control (re-register as owner) of the

interface for a certain number of deciseconds.

Notes: The purpose of this request is to provide a means for a new

device/interface driver to "wrest" control from an existing

("old") device/interface driver.  Generally, the only time

this might be needed is when a vendor-specific or product-

specific driver wants to take control of an device/interface

that is already being controlled by a general-purpose driver.

This request would not typically be used by "normal" device

drivers.

BIOS-LEVEL USB API 0.05            Page 128 of 168

Before the "old" interface owner returns from the

notification, it must issue appropriate requests back to the

host driver to completely relinquish control of the

interface.  It must cancel all scheduled bus transactions

relating to the interface, unregister as the interface owner,

and initiate an internal don't look timeout process.  The

"old" interface owner must not attempt reset the device as a

"shortcut" to relinquishing control.

During the don't look time, the "old" interface owner must

not look for any new interfaces.  If the "new" interface

owner must reset the device to initialize it properly, the

"old" interface owner will not be able to recognize the re-

enumerated device as the one it's supposed to be "ignoring".

The "old" interface owner cannot simply ignore what the

notification is requesting in an attempt to try and maintain

control of the interface.  The "old" interface owner must

immediately relinquish control of the interface and not try

to regain control during the requested don't look time.

I14RTimeout should be kept as small as possible.  If the

"new" interface owner will not need to reset the device,

I14RTimeout should only need to be 1 or 2 deciseconds.  If

the device does need to be reset, I14RTimeout will probably

need to be significantly longer (something on the order of 20

deciseconds should be sufficient).

If I14RTimeout is 0, a "default" value of 20 deciseconds will

be sent to the "old" interface owner in the DH register.

Although I14RTimeout is designated as a 16-bit word, for this

particular request ("Unregister Current Interface Owner and

Don't Look"), the most significant byte of the word is

ignored, and only the least significant byte of the word is

used.  This means that the maximum number of deciseconds

allowed for the don't look timeout value is 255 deciseconds.

BIOS-LEVEL USB API 0.05            Page 129 of 168

Get Alternate Interface Information

�����������������������������������

Class:       Alternate Interface

Return Type: Immediate

Entry:

I14RRequestType   = 71h (I14RRTGetAltIntfInfo)

I14RHostIndex     = Host Index Device is attached to (0-15)

I14RDeviceAddress = Address of Device Interface is on (2-127)

I14RInterfaceNum  = Interface Number the Alternate Interface is

associated with

I14RAltInterface  = Alternate Interface Number to get Info for

Return (Success):

CF = Clear

AX = 0

BL = Interface Class

BH = Interface SubClass

CL = Interface Protocol

CH = Alternate Interface Bitmap

Bit 0 (01h) = Set if Alternate Interface is Selected

Bit 1 (02h) = Set if Interface is Owned

Bits 2-7    = Reserved (0)

DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: The host driver must monitor the appropriate control requests

in "real time" so that it can process this request as an

immediate return.  The host must NOT send a control packet

across the bus requesting status(es) directly from the device

before processing this request.

Note that if an interface has multiple alternate interfaces

"underneath" it, it is possible for bit 1 of the CH register

to be set (indicating that the interface is owned) even if

bit 0 is not set (indicating that alternate interface is not

the one currently selected).  An unselected alternate

interface is effectively irrelevant (it cannot be used for

anything), even though it may be "owned" because the

interface "above" it is owned.

BIOS-LEVEL USB API 0.05            Page 130 of 168

Get Current DataX Value

�����������������������

Class:       End Point

Return Type: Immediate

Entry:

I14RRequestType    = 81h (I14RRTGetDataX)

I14RHostIndex      = Host Index Device is attached to (0-15)

I14RDeviceAddress  = Address of Device Interface is on (2-127)

I14REndPoint       = End Point to get DataX value of (1-15)

(Bulk or Interrupt EndPoint)

I14RFlags:

I14RFlagIn (01h) = Set if End Point Direction is In

= Clear if End Point Direction is Out

Return (Success):

CF = Clear

AX = 0

BL = Current DataX value for End Point

00h = Data0

01h = Data1

BH = FFh

CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: DataX (Data0 and Data1) is a flow control mechanism used by

the USB architecture when sending multiple packets to the

same end point.  Read the USB specifications if you need

further implementation details.

The direction associated with the end point is embedded in

bit 0 (I14RFlagIn) of the I14RFlags register.  This is NOT

the same format used in the end point descriptor (bit 7 of

the end point number).

The host driver normally handles all DataX processing

automatically.  This request, in addition to the "Increment

(Toggle) DataX Value" request (page 132), allows you to

manually change the current DataX value if the end point

requires it for some reason (such as if the device does not

respond appropriately to "Clear End Point Feature: Halt"

control requests).

BIOS-LEVEL USB API 0.05            Page 131 of 168

Increment (Toggle) DataX Value

������������������������������

Class:       End Point

Return Type: Immediate

Entry:

I14RRequestType    = 88h (I14RRTIncDataX)

I14RHostIndex      = Host Index Device is attached to (0-15)

I14RDeviceAddress  = Address of Device Interface is on (2-127)

I14REndPoint       = End Point to change DataX value of (1-15)

(Bulk or Interrupt EndPoint)

I14RFlags:

I14RFlagIn (01h) = Set if End Point Direction is In

= Clear if End Point Direction is Out

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: DataX (Data0 and Data1) is a flow control mechanism used by

the USB architecture when sending multiple packets to the

same end point.  Read the USB specifications if you need

further implementation details.

The direction associated with the end point is embedded in

bit 0 (I14RFlagIn) of the I14RFlags register.  This is NOT

the same format used in the end point descriptor (bit 7 of

the end point number).

The host driver normally handles all DataX processing

automatically.  This request, in addition to the "Get Current

DataX Value" request (page 131), allows you to manually

change the current DataX value if the end point requires it

for some reason (such as if the device does not respond

appropriately to "Clear End Point Feature: Halt" control

requests).

The host driver monitors all "Clear End Point Feature: Halt"

control requests, and if they are successful, automatically

resets the end point to Data0.

BIOS-LEVEL USB API 0.05            Page 132 of 168

Changing the DataX value should only be performed on end

points that process bulk and interrupt transactions.

Isochronous transactions do not use DataX flow control, and

control transactions always initialize at Data0.

BIOS-LEVEL USB API 0.05            Page 133 of 168

Schedule Isochronous Transaction

��������������������������������

Class:       Packet

Return Type: Schedule

Entry:

I14RRequestType    = 94h (I14RRTDoIsoch)

I14RHostIndex      = Host Index Device is attached to (0-15)

I14RDeviceAddress  = Device Address to send Transaction to (2-127)

I14REndPoint       = End Point to send Transaction to (0-15)

I14RPeriodicity    = # of Frames before schedule repeats (1-1024)

(Number of entries in the

I14RIsochSchedAddr Table)

I14RDataAddress    = Memory Address associated with first

scheduled Isochronous Frame

(32-bit Physical or Segment:Offset)

I14RFrameIndex     = Starting Frame Index to use (0-1023)

(only valid if I14RFlagSpecificFrame set)

I14RIsochSchedAddr = Address where Isochronous Schedule Table

is located (Segment:Offset)

I14RCallBackAddr   = Notification Call Back Address (CS:IP)

(Packet Owner)

I14RUserPktID      = Value to return in BX on Notifications

I14RFlags:

I14RFlagIn             (01h) = Set if Data Direction = In

(Device to Host)

= Clear if Data Direction = Out

(Host to Device)

I14RFlagSpecificFrame  (40h) = Set if the Schedule should start

at a specific Frame Index (the

one in I14RFrameIndex)

= Clear if Host Driver should

automatically select the

starting Frame Index

I14RFlagAddrIsPhysical (80h) = Set if I14RDataAddress is a

32-bit Physical Address

= Clear if I14RDataAddress is a

Segment:Offset Address

Return (Success):

CF = Clear

AX = 0

BX = Starting Frame Index (0-1023)

CX = Transaction Handle

DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

BIOS-LEVEL USB API 0.05            Page 134 of 168

Owner Notifications:

AX = (see Status Codes Table on page 14)

BX = I14RUserPktID of Packet Owner

CX = Actual Number of Bytes Transferred by the Packet

DL = Packet Sequence Number (0 = First Packet in Schedule, etc.)

DH = 0

The packet owner is notified of the status of each packet

scheduled to be transferred across the bus.

Notes: The isochronous schedule table (the table stored at

I14RIsochschedAddr) is a list of words containing the number

of bytes to transfer in each frame.  Each entry in the table

must be between 0 and 1023.

There can be a maximum of 256 non-zero entries in the

isochronous schedule table (entries indicating that at least

one byte is to be transmitted in the frame).  The maximum

number of bytes that can be transmitted in an isochronous

period is 261,888 (1023 bytes/frame * 256 frames).

Isochronous transactions, unlike other types of packet

transactions, do not naturally provide constant status

"feedback".  An isochronous transaction may need another

"corresponding" transaction (like a periodic interrupt) in

order to maintain synchronization with the bus.

The individual packet status notifications received by the

packet owner will not necessarily be in the same order that

the packets are scheduled.  The packet sequence number (the

DL register on packet owner notifications) can be used to

"reorganize" the packets into the correct order if needed.

The transaction handle (the value returned in CX) must be

"returned" (in I14RRequestHandle) in the "Close Transaction

Handle" request (page 147).

BIOS-LEVEL USB API 0.05            Page 135 of 168

An example request might look something like this:

I14RRequestType   =   94h ;I14RRTDoIsoch

I14RFlags         =   41h ;Data Direction = In

;Start at the Frame Index

;  in I14RFrameIndex

;Data Address is Segment:Offset

I14RHostIndex     =    0  ;Host Index 0

I14RDeviceAddress =    6  ;Device Address 6

I14REndPoint      =    1  ;End Point 1

I14RPeriodicity   =   10h ;Schedule repeats after

;  16 Frames

;There are 16 entries in

;  the Schedule Table

I14RFrameIndex    =    3  ;Start at Frame Index 3

I14RCallBackAddr   = 2222:0200h ;Notification Address

I14RUserPktID      =      0000h ;Value to return in BX

;  on Notifications

I14RDataAddress    = 2222:1000h ;Segment:Offset where

;  first byte of data

;  will be stored

I14RIsochSchedAddr = 2222:1200h ;Segment:Offset where

;  Schedule Table

;  is stored

Data stored at 2222:1200h (the Schedule Table):

DW 0020h, 0000h, 0000h, 0000h ;32, 0, 0, 0

DW 0010h, 0000h, 0000h, 0000h ;16, 0, 0, 0

DW 0010h, 0000h, 0000h, 0000h ;16, 0, 0, 0

DW 0030h, 0000h, 0000h, 0000h ;48, 0, 0, 0

This example request would set up an isochronous

transaction on the bus, which would repeat every 16

frames.  At frame index 3 (sequence 0), 32 bytes would be

scheduled to transfer from host 0, device 6, endpoint 1 to

memory address 2222:1000h.  At frame index 7 (sequence 1),

16 bytes would be scheduled to transfer from host 0,

device 6, endpoint 1 to memory address 2222:1020h.  At

frame index 11 (sequence 2), 16 bytes would be scheduled

to transfer from host 0, device 6, endpoint 1 to memory

address 2222:1030h.  At frame index 15 (sequence 3), 48

bytes would be scheduled to transfer from host 0, device

6, endpoint 1 to memory address 2222:1040h.  At frame

index 19, the schedule will start all over again (sequence

0), and 32 bytes would be scheduled to transfer from host

0, device 6, endpoint 1 to memory address 2222:1000h.  The

schedule will continue indefinitely until the host

receives a "Close Transaction Handle" request, or until

device 6 is disconnected from host 0.

BIOS-LEVEL USB API 0.05            Page 136 of 168

Schedule Periodic Interrupt Transaction

���������������������������������������

Class:       Packet

Return Type: Schedule

Entry:

I14RRequestType    = 95h (I14RRTDoInterruptPer)

I14RHostIndex      = Host Index Device is attached to (0-15)

I14RDeviceAddress  = Device Address to send Transaction to (1-127)

I14REndPoint       = End Point to send Transaction to (0-15)

I14RPeriodicity    = Number of Frames between Interrupts (1-1024)

I14RDataAddress    = Memory Address where Data is/will be stored

(32-bit Physical or Segment:Offset)

I14RDataSize       = Bytes to Transfer each Interrupt (0-65535)

(see Notes regarding upper limit)

I14RFrameIndex     = Starting Frame Index to use (0-1023)

(only valid if I14RFlagSpecificFrame set)

I14RCallBackAddr   = Notification Call Back Address (CS:IP)

(Packet Owner)

I14RUserPktID      = Value to return in BX on Notifications

I14RFlags:

I14RFlagIn             (01h) = Set if Data Direction = In

(Device to Host)

= Clear if Data Direction = Out

(Host to Device)

I14RFlagSpecificFrame  (40h) = Set if Interrupts should start

at a specific Frame Index (the

one in I14RFrameIndex)

= Clear if Host Driver should

automatically select the

starting Frame Index

I14RFlagAddrIsPhysical (80h) = Set if I14RDataAddress is a

32-bit Physical Address

= Clear if I14RDataAddress is a

Segment:Offset Address

Return (Success):

CF = Clear

AX = 0

BX = DX = FFFFh

CX = Transaction Handle

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

BIOS-LEVEL USB API 0.05            Page 137 of 168

Owner Notifications:

AX = (see Status Codes Table on page 14)

BX = I14RUserPktID of Packet Owner

CX = Actual Number of Bytes Transferred by the Packet

DX = 0

The packet owner is notified of the status of each packet

scheduled to be transferred across the bus.

Notes: Unlike most other types of packet transactions, a periodic

interrupt can be scheduled to go to device address 1 (the

root hub).  The root hub is a "virtual" device that does not

actually exist on the bus, so any packets scheduled to go to

the root hub will fail (notifications will contain a non-zero

status code in AX).  Such packet transactions can be used

simply to generate periodic interrupts that control timing

(they will never actually return any valid USB device data).

I14RDataSize is listed as allowing any number between 0 and

65535, but the upper limit is actually dependent on several

factors, including the speed of the device and the

characteristics of the endpoint.  The absolute maximum size

of a periodic interrupt transaction to a low-speed device is

48 bytes, and to a full-speed device is 1043 bytes.

If I14RDataSize is larger than the maximum packet size

allowed by the endpoint, the host driver will automatically

"divide" the transaction into smaller, appropriately-sized

packets.  Owner notifications will always be processed as if

there were a single packet scheduled on the bus.

If I14RFlagSpecificFrame is set, the periodicity of the

interrupt will be maintained religiously by the host driver.

If the bus is heavily loaded, this may result in packets

becoming "lost" (never actually being sent across the bus,

resulting in a status code which includes 0008h, or

TDStsOverDue).  If I14RFlagSpecificFrame is not set, and the

bus is heavily loaded, the host driver may periodically

"slide" the packet back and forth in the schedule slightly to

decrease the odds of it becoming "lost".

While it is theoretically possible to schedule a periodic

interrupt with a periodicity of 1 frame, it may not actually

be possible to do (at least not reliably).  The minimum

recommended periodicity is 2 frames.

The transaction handle (the value returned in CX) must be

"returned" (in I14RRequestHandle) in the "Close Transaction

Handle" request (page 147).

BIOS-LEVEL USB API 0.05            Page 138 of 168

Schedule Control (Setup) Transaction

������������������������������������

Class:       Packet

Return Type: Schedule

Entry:

I14RRequestType   = 96h (I14RRTDoControl)

I14RHostIndex     = Host Index Device is attached to (0-15)

I14RDeviceAddress = Device Address to send Transaction to (2-127)

I14REndPoint      = End Point to send Transaction to (0-15)

I14RTimeout       = Transaction Timeout value (deciseconds)

(0-18000)

I14RDataAddress   = Memory Address where Data is/will be stored

(32-bit Physical or Segment:Offset)

I14RCallBackAddr  = Notification Call Back Address (CS:IP)

(Transaction Owner)

I14RUserPktID     = Value to return in BX on Notification

I14RSetupReqData  = 8-byte Control Setup Packet to send to Device

(contains embedded Direction & Data Size)

I14RFlags:

I14RFlagNoRetries      (10h) = Set if unsuccessful

Transactions should not be

not automatically retried

= Clear if unsuccessful

Transactions should be

Automatically retried

I14RFlagShortPktOK     (20h) = Set if short data packet

errors should not be retried

= Clear if short data packet

errors should be retried

I14RFlagAddrIsPhysical (80h) = Set if I14RDataAddress is a

32-bit Physical Address

= Clear if I14RDataAddress is a

Segment:Offset Address

Return (Success):

CF = Clear

AX = 0

BX = FFFFh

CX = Transaction Handle

DL = Closure ID (non-zero)

DH = FFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

BIOS-LEVEL USB API 0.05            Page 139 of 168

Owner Notification:

AX = (see Status Codes Table on page 14)

BX = I14RUserPktID of Transaction Owner

CX = Actual Number of Bytes Transferred by the Transaction

DX = 0

The transaction owner is notified of the status of the

transaction after it has been completed.

Notes: The 8-byte setup control packet (I14RSetupReqData) contains

embedded the direction (in/out) & data size (bytes) of the

actual data (if any) to be transferred to/from the device

after the setup packet has been transferred.  Therefore,

I14RDataSize & I14RFlagIn are not used.

I14RDataAddress contains the address of the actual data (if

any) to be transferred after the setup packet has been

transferred.

If the actual data size is larger than the maximum packet

size allowed by the endpoint, the host driver will

automatically "divide" the transaction into smaller,

appropriately-sized packets.

The value in I14RTimeout is used to keep the transaction from

"stalling out" due to an unresponsive device.  If the

transaction has not completed before the timeout period has

expired, the transaction will automatically be canceled by

the host driver and a status code which includes 0004h

(TDStsTimeout) will be returned.

If I14RFlagNoRetries is clear, the control transaction will

be retried as many as 8 times before finally declaring the

transaction to be in error.  There is no time delay between

each retry.  The error code returned will correspond to the

status of the last retry.

The value in I14RTimeout is the time allotted for each retry,

not the total time for all retries combined.  For example,

using the default values of 8 retries and 10 deciseconds (one

second) per attempt, a control Transaction that fails each

time due to a Timeout will require a minimum of 9 seconds to

complete.

The clock that controls the timeout stops counting down while

the host is stopped/halted.

If I14RTimeout is 0, a "default" value of 10 deciseconds will

be used.  The maximum value allowed for I14RTimeout is 18000

deciseconds (approximately 1/2 hour).

BIOS-LEVEL USB API 0.05            Page 140 of 168

The only type of transaction that can be automatically

retried, in it entirety, is a control transaction: entire

bulk, interrupt, and isochronous transactions are not retried

automatically.

If I14RFlagShortPacketOK is set, the transaction will not be

considered in error if the only error is a short packet.

This can be useful, for example, when requesting a string

descriptor from a device, where the actual length of the

descriptor is unknown until after it has already been

downloaded.

If the status code returned at the end of the transaction

with the TDStsControlSetup bit (1000h) set, it means that the

error occurred while the transaction was still in the setup

stage, and the actual data stage was never reached.  The most

significant implication of this is that the value returned in

the CX register (the actual number of bytes transferred by

the transaction) is the number of bytes of the 8-byte setup

packet that were transferred, not the number of bytes of

"real" data that were transferred.

If a control transaction is issued to end point 0 of a

device, and there is already a control transaction in

progress to the same end point (end point 0) of the same

device, the host driver will automatically place the new

request "On Hold" (in a queue awaiting transmission onto the

bus).  When the original (older) control transaction is

completed, the newer control transaction will be

automatically sent across the bus.  This is done because

endpoint 0 is common to all interfaces of a device, but only

one control transaction at a time can typically be processed

by a device.  Interface drivers are independent and not

required to communicate and coordinate schedules with one

another, so this functionality is automatically provided by

the host driver.

The host driver needs to "monitor" certain types of control

requests for internal tracking purposes.  Examples of

requests that need to be monitored are "Set Device

Configuration", "Set Alternate Interface", and "Clear End

Point Feature: Halt".

The host driver will reject all control requests to change a

device's address.

BIOS-LEVEL USB API 0.05            Page 141 of 168

The transaction handle and closure ID (the values returned in

CX and DL) must be "returned" (in I14RRequestHandle and

I14RCloseID, respectively) in the "Close Transaction Handle"

request (page 147).  There should almost never be a need to

close a control transaction prematurely, however, as long as

I14RTimeout was set appropriately.

The closure ID (the value returned in DL) will never be 0.

BIOS-LEVEL USB API 0.05            Page 142 of 168

Schedule Bulk Transaction

�������������������������

Class:       Packet

Return Type: Schedule

Entry:

I14RRequestType    = 97h (I14RRTDoBulk)

I14RHostIndex      = Host Index Device is attached to (0-15)

I14RDeviceAddress  = Device Address to send Transaction to (2-127)

I14REndPoint       = End Point to send Transaction to (0-15)

I14RTimeout        = Transaction Timeout value (deciseconds)

(0-18000)

I14RDataAddress    = Memory Address where Data is/will be stored

(32-bit Physical or Segment:Offset)

I14RDataSize       = Bytes to Transfer (0-65535)

I14RCallBackAddr   = Notification Call Back Address (CS:IP)

(Transaction Owner)

I14RUserPktID      = Value to return in BX on Notification

I14RFlags:

I14RFlagIn             (01h) = Set if Data Direction = In

(Device to Host)

= Clear if Data Direction = Out

(Host to Device)

I14RFlagAddrIsPhysical (80h) = Set if I14RDataAddress is a

32-bit Physical Address

= Clear if I14RDataAddress is a

Segment:Offset Address

Return (Success):

CF = Clear

AX = 0

BX = FFFFh

CX = Transaction Handle

DL = Closure ID (non-zero)

DH = FFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Owner Notification:

AX = (see Status Codes Table on page 14)

BX = I14RUserPktID of Transaction Owner

CX = Actual Number of Bytes Transferred by the Transaction

DX = 0

The transaction owner is notified of the status of the

transaction after it has been completed.

BIOS-LEVEL USB API 0.05            Page 143 of 168

Notes: If I14RDataSize is larger than the maximum packet size

allowed by the endpoint, the Host Driver will automatically

"divide" the transaction into smaller, appropriately-sized

packets.  The owner notification will always be processed as

if the transaction were a single packet scheduled on the bus.

The value in I14RTimeout is used to keep the transaction from

"stalling out" due to an unresponsive device.  If the

transaction has not completed before the timeout period has

expired, the transaction will automatically be canceled by

the host driver and a status code which includes 0004h

(TDStsTimeout) will be returned.

The clock that controls the timeout stops counting down while

the host is stopped/halted.

If I14RTimeout is 0, a "default" value of 50 deciseconds will

be used.  The maximum value allowed for I14RTimeout is 18000

deciseconds (approximately 1/2 hour).

The transaction handle and closure ID (the values returned in

CX and DL) must be "returned" (in I14RRequestHandle and

I14RCloseID, respectively) in the "Close Transaction Handle"

request (page 147).  There should almost never be a need to

close a bulk transaction prematurely, however, as long as

I14RTimeout was set appropriately.

The closure ID (the value returned in DL) will never be 0.

BIOS-LEVEL USB API 0.05            Page 144 of 168

Schedule One-Time Interrupt Transaction

���������������������������������������

Class:       Packet

Return Type: Schedule

Entry:

I14RRequestType    = 98h (I14RRTDoInterrupt1T)

I14RHostIndex      = Host Index to use (0-15)

I14RPeriodicity    = Number of Frames to Delay (1-65535)

I14RCallBackAddr   = Notification Call Back Address (CS:IP)

(Transaction Owner)

I14RUserPktID      = Value to return in BX on Notification

Return (Success):

CF = Clear

AX = 0

BX = FFFFh

CX = Transaction Handle

DL = Closure ID (non-zero)

DH = FFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Owner Notification:

AX = 0000h (TDStsOK)

BX = I14RUserPktID of Transaction Owner

CX = DX = 0

The transaction owner is notified after the transaction has been

completed.

Notes: This request effectively issues a "background delay", which

can be used as a very effective way to control timing of

background processes.

It is not possible to send data across the bus with a one-

time interrupt transaction.  It is only possible to control

timing.

The resolution of the delay timer is one USB frame

(approximately one millisecond).  The maximum delay allowed

is 65535 USB frames, which is approximately 65.535 seconds.

BIOS-LEVEL USB API 0.05            Page 145 of 168

Internally, the host controller driver issues a packet

request on the bus to device address 1 (the Root Hub), end

point 0.  Therefore, device address 1 and end point 0 must be

provided In the "Get Status of Packet Transaction" call (page

150) when requesting the status of a one-time interrupt

transaction.

The transaction handle and closure ID (the values returned in

CX and DL) must be "returned" (in I14RRequestHandle and

I14RCloseID, respectively) in the "Close Transaction Handle"

request (page 147).

The closure ID (the value returned in DL) will never be 0.

BIOS-LEVEL USB API 0.05            Page 146 of 168

Close Transaction Handle

������������������������

Class:       Packet

Return Type: Immediate

Entry:

I14RRequestType   = 9Ch (I14RRTCloseHandle)

I14RHostIndex     = Host Index Transaction is on (0-15)

I14RCloseID       = Closure ID of Transaction to Close

(only needed for Bulk, Control, and

One-Time Interrupt Transactions)

I14RRequestHandle = Handle number of Transaction to Close

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This request closes (cancels prematurely) all types of packet

transactions (isochronous, periodic interrupt, control, bulk,

and one-time interrupt).

The transaction handle (in I14RRequestHandle) is the value

that was returned in the CX register during the request that

initiated the transaction.

The closure ID (in I14RCloseID) is the value that was

returned in the DL register during the request that initiated

the transaction.  This only applies to bulk, control (setup),

and one-time interrupt packet transactions.  Isochronous and

periodic interrupt transactions do not require closure ID's.

Handle numbers can be reused by a host driver immediately

after a transaction is closed, and a host driver

automatically closes certain types of transactions upon

completion.  By requiring both a closure ID and a handle

number in order to close a transaction, the host driver can

be certain that a request to close a handle is truly being

issued by the correct owner.

BIOS-LEVEL USB API 0.05            Page 147 of 168

When a device is disconnected or reset, all packet

transactions currently in progress or scheduled to the device

are AUTOMATICALLY removed from the schedule, immediately

after being issued a TD status code of TDStsDvcRemoved.  This

request must NEVER be issued by a device driver after being

notified that the device has been disconnected, or it could

adversely affect a device other than the one intended.

BIOS-LEVEL USB API 0.05            Page 148 of 168

Change Periodicity of Periodic Interrupt

����������������������������������������

Class:       Packet

Return Type: Immediate

Entry:

I14RRequestType   = 9Dh (I14RRTChangeIntPer)

I14RHostIndex     = Host Index Transaction is on (0-15)

I14RRequestHandle = Handle number of Transaction to Change

(must be a Periodic Interrupt Handle)

I14RPeriodicity   = Number of Frames between Interrupts (1-1024)

(new value to replace current Periodicity)

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This request provides a simple way to change the "sampling

rate" of a periodic interrupt.  Without this request,

changing the periodicity would require closing the existing

periodic interrupt transaction, and then scheduling a new

transaction.

The transaction handle (in I14RRequestHandle) is the value

that was returned in the CX register during the original

"Schedule Periodic Interrupt Transaction" request (page 137).

BIOS-LEVEL USB API 0.05            Page 149 of 168

Get Status of Packet Transaction

��������������������������������

Class:       Packet

Return Type: Immediate

Entry:

I14RRequestType   = 9Fh (I14RRTGetTransStatus)

I14RHostIndex     = Host Index Transaction is on (0-15)

I14RDeviceAddress = Device Address that Transaction was

sent to (1-127)

I14REndPoint      = End Point that Transaction was sent to (0-15)

I14RRequestHandle = Handle number of Transaction to get Status of

Return (Success):

CF = Clear

AX = 0

BH = 0

BL = Flags/Special Codes

Bulk:         BL = 0

Control:      BL = Stage & Automatic Retry Status

Low Nibble = Current Stage

0 = On Hold

1 = Setup Stage

2 = Data Stage

3 = Status Stage

4-15 = Reserved (Unused)

High Nibble = Number of times the

Transaction will be automatically

attempted (retried) if the current

attempt is unsuccessful (0-7)

Isochronous:  BL = High Byte of Number of Data Bytes

remaining to be sent in current

Transaction Period (total number of

data bytes is BL:DX)

Periodic Int: BL = Overdue Status

Bits 0-6    = Reserved (0)

Bit 7 (80h) = Set if Transaction is

Overdue

One-Time Int: BL = Overdue Status

Bits 0-6    = Reserved (0)

Bit 7 (80h) = Set if Transaction is

Overdue

BIOS-LEVEL USB API 0.05            Page 150 of 168

CX = Approximate time left until Transaction is complete

Bulk:         CX = Deciseconds remaining until

Transaction Timeout

Control:      CX = Deciseconds remaining until

Transaction Timeout

Isochronous:  CX = Frames remaining in Transaction Period

Periodic Int: CX = Frames remaining in Transaction Period

One-Time Int: CX = Frames remaining until Transaction

is complete

DX = Approximate number of bytes remaining in Transaction

Bulk:         DX = Bytes remaining to be sent in Transaction

Control:      DX = Bytes remaining to be sent in Data Stage

of Transaction

Isochronous:  DX = Low Word of Number of Data Bytes

remaining to be sent in current

Transaction Period (total number of

data bytes is BL:DX)

Periodic Int: DX = Bytes remaining to be sent in current

Transaction Period

One-Time Int: DX = 0

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This request provides the method to monitor the progress of a

packet-based Transaction that is being processed in the

background (Bulk, Control, Isochronous, Periodic Interrupt,

or One-Time Interrupt).  There is no method provided to

monitor other types of Transactions (such as those with a

Large Return).

The transaction handle (in I14RRequestHandle) is the value

that was returned in the CX register during the request that

initiated the transaction.

The Device Address (I14RDeviceAddress) and End Point

(I14REndPoint) must be the same as those provided during the

request that initiated the transaction.  These must be

provided to ensure that only the requester of original

transaction can request the status.  If the transaction is a

one-time interrupt, I14RDeviceAddress must be 1, and

I14REndPoint must be 0.

BIOS-LEVEL USB API 0.05            Page 151 of 168

Values returned in CX (time), DX (bytes), or BL:DX (bytes)

are approximate, not exact.  The transaction is processed by

the hardware as a background process, and the status as

provided by this request is simply an attempt by the software

to take an instantaneous "snapshot" of the hardware at a

particular instant in time.

A control transaction is given a status of "On Hold" if it is

directed at end point 0, and there is already another control

transaction scheduled to end point 0 of the same device.  "On

Hold" indicates that the control transaction is in queue

awaiting the previous (older) control transaction to

complete.  When the previous control transaction is complete,

the newer control transaction will be automatically sent

across the bus (the status of the newer transaction will

change from "On Hold" to "Setup Stage").  This is done

because endpoint 0 is common to all interfaces of a device,

but only one control transaction at a time can typically be

processed by a device.  Interface drivers are independent and

not required to communicate and coordinate schedules with one

another, so this functionality is automatically provided by

the host driver.

A periodic interrupt or one-time interrupt transaction is

considered overdue when it is scheduled to occur at some

particular time but is not processed properly by the host

hardware for some reason.  This can happen, for instance, if

the host hardware does not process hardware interrupts

(IRQ's) properly, or if the bus is near capacity.

Isochronous packets can fail for many of the same reasons as

periodic and interrupt transactions, but are never considered

"overdue" simply due to the definition of isochronous

transactions.

The units returned for time values (in the CX register) are

deciseconds for bulk and control transactions, and USB frame

durations (milliseconds) for isochronous, periodic interrupt,

and one-time Interrupt transactions.

The value returned in the CX register (deciseconds remaining

until transaction timeout) for bulk and control transactions

is not necessarily the time remaining in the transaction.  If

the transaction completes in the hardware before the timeout

period expires, whether the transaction is successful or

unsuccessful, the transaction is consider complete.

BIOS-LEVEL USB API 0.05            Page 152 of 168

The value returned in the DX register for control

transactions (number of bytes remaining) is only the number

of bytes remaining to be sent in the data stage of the

transaction.  The setup stage of a control transaction is

always exactly 8 bytes, but the count of the bytes in the

setup stage is not monitored or returned by this request.

BIOS-LEVEL USB API 0.05            Page 153 of 168

Large Call Completed

��������������������

Class:       Miscellaneous

Return Type: Immediate

Entry:

I14RRequestType      = A1h (I14RRTLargeCallDone)

I14RHostIndex        = Host Index Large Call is on (0-15)

I14RRequestHandle    = Large Call Index (in Low Byte)

(contents of DL register when original

Large Call Notification was issued)

I14RLargeCallRtnCode = Error Code word

(0000h=TDStsOK or 8000h=TDStsLargeCallErr)

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This request is how an interface owner signals the end of a

large transaction back to the host driver.  A large

transaction originally comes into the host driver via a call

from another piece of software.  The host driver then issues

a large call notification to the appropriate interface owner.

The large call notification includes a large call index,

provided by the host driver to the interface owner in the DL

register during the notification.  The purpose of the large

call notification is to temporarily pass complete control of

the large transaction over to the interface owner.  After the

interface owner has completed processing the large call, it

MUST issue this request back to the host driver indicating

that the large call has been completed.  The interface owner

must provide back to the host driver the original large call

index (in the low byte of I14RRequestHandle), along with a

success/failure indication (in the I14RLargeCallRtnCode

word).  The host driver will then notify the original large

transaction caller of the transaction status.

There are several types of requests that are classified as

large transactions, most of them related to hubs.  For

example, in order to enable, disable, reset, suspend, resume,

power on, power off, or power reset a device, it is required

that a large transaction be issued to the owner of the hub

that the device is attached to.

BIOS-LEVEL USB API 0.05            Page 154 of 168

Beep the Speaker

����������������

Class:       Miscellaneous

Return Type: Immediate

Entry:

I14RRequestType   = AFh (I14RRTBeepSpeaker)

I14RHostIndex     = Host Index (0-15, 255 = Any Host)

I14RBeepFrequency = Frequency to Beep with (Hertz) (20-20000)

I14RTimeout       = Duration of Beep (deciseconds) (0-18000)

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This request is provided for other USB-related background

programs (like TSR's) to easily indicate error conditions by

beeping the speaker.  By providing this feature in the host

driver, each individual device/interface driver does not need

to contain code required to beep the speaker.

The value in I14RTimeout will be the approximate duration of

the beep, not necessarily the precise duration.

The host driver may not necessarily use the USB hardware to

control the duration of the beep.

This request generates sounds using the computer's internal

speaker.  It does not generate sounds through the high-

quality sound system installed on most modern computers.

Whether the request is issued to an individual host or to any

host, the speaker will only beep one time.  It does not beep

once for each host driver that is installed.

If the host driver detects that the internal speaker is

already being used (by another program) when it tries to

start beeping, the host driver will simply cancel the beep

process without notification.

If I14RBeepFrequency is 0, a "default" value of 1000 Hertz

will be used.  If I14RTimeout is 0, a "default" value of 3

deciseconds will be used.

BIOS-LEVEL USB API 0.05            Page 155 of 168

The maximum value allowed for I14RTimeout is 18000

deciseconds (approximately 1/2 hour), though a beep will

typically only last a few deciseconds.

If this request is issued with I14RHostIndex = 255 (any

host), the first host driver to receive the request will

simply process the request as if it had been issued with that

host driver's specific host index.  It will not "pass on" the

request to other host driver(s).

BIOS-LEVEL USB API 0.05            Page 156 of 168

A New Host Driver Has Been Installed

������������������������������������

Class:       InterHost

Return Type: Immediate

Entry:

I14RRequestType = E1h (I14RRTHostAdded)

I14RHostIndex   = Host Index (0-15, 255 = All Hosts)

(existing Host Index(es))

I14RConfigValue = New Host Index (0-15)

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This request is issued by a new host driver during its

initialization process if other host driver(s) were installed

before it was.

The new host driver must assign itself a unique host index,

one which has not already been assigned to an existing host

driver.  It can determine which indexes have already been

assigned by issuing a "Get Host Hardware Information" request

(page 44) with I14RHostIndex = 255 (all hosts).

The new host driver must inform ALL existing host drivers of

its presence.  If it does not issue this request with

I14RHostIndex = 255 (all hosts), it must issue separate

requests to each existing host driver individually.

If this request is issued with I14RHostIndex = 255 (all

hosts), the first existing host driver to receive the request

will "pass on" the request to all existing host drivers

individually.

BIOS-LEVEL USB API 0.05            Page 157 of 168

An Existing Host Driver Has Been Removed

����������������������������������������

Class:       InterHost

Return Type: Immediate

Entry:

I14RRequestType = E2h (I14RRTHostDeleted)

I14RHostIndex   = Host Index (0-15, 255 = All Hosts)

(existing Host Index(es))

I14RConfigValue = Host Index being Removed (0-15)

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This request is issued by a host driver during its shutdown

process if other host driver(s) will still be installed after

it is gone.

If the host driver being removed is the last (only) host

driver remaining, it cannot be uninstalled if there are

remaining registered device 0 owners.  Any program that

requires a host driver to be installed in order to keep from

"blowing up" must register as a device 0 owner.

The host driver being removed must inform ALL remaining host

drivers of its removal.  If it does not issue this request

with I14RHostIndex = 255 (all hosts), it must issue separate

requests to each host driver individually.

If this request is issued with I14RHostIndex = 255 (all

hosts), the first host driver to receive the request will

"pass on" the request to all host drivers individually.

BIOS-LEVEL USB API 0.05            Page 158 of 168

InterHost Register As Device 0 Owner

������������������������������������

Class:       InterHost

Return Type: Immediate

Entry:

I14RRequestType   = E3h (I14RRTRegDvc0Host)

I14RHostIndex     = Host Index (0-15)

I14RDeviceAddress = 0 (Newly Attached Devices)

I14RVendorID      = Dvc Vendor ID (FFFFh or 0000h = Don't Care)

I14RProductID     = Dvc Product ID (FFFFh or 0000h = Don't Care)

I14RDvcClass      = Device Class of Device (FFh = Don't Care)

I14RDvcSubClass   = Device SubClass of Device (FFh = Don't Care)

I14RDvcProtocol   = Device Protocol of Device (FFh = Don't Care)

I14RIntfClass     = Interface Class of Alternate Interface

(FFh = Don't Care)

I14RIntfSubClass  = Interface SubClass of Alternate Interface

(FFh = Don't Care)

I14RIntfProtocol  = Interface Protocol of Alternate Interface

(FFh = Don't Care)

I14RRequestHandle = Device 0 Owner Handle Number

(Device 0 Ownership Table Index)

(Assigned by the first Host Driver to

receive the "Register As Device 0 (New

Interface) Owner" Request

I14RCallBackAddr  = Notification Call Back Address (CS:IP)

(Device 0 Owner)

I14RUserPktID     = Value to return in BX on Notifications

Return:

CF = Clear

AX = BX = CX = DX = 0

Owner Notifications:

AX = 0001h (OwnerCallNewDvc)

BX = I14RUserPktID of Device 0 Owner

CL = Host Index (0-15)

CH = Device Address of Newly Attached Device (2-127)

DL = Interface Number that matching Alternate Interface is on

DH = Matching Alternate Interface Number

Notifies all registered device 0 owners whose search criteria

match the characteristics of the newly attached device

(I14RVendorID, I14RProductID, I14RDvcClass, I14RDvcSubClass,

I14RDvcProtocol, I14RIntfClass, I14RIntfSubClass, and

I14RIntfProtocol).

BIOS-LEVEL USB API 0.05            Page 159 of 168

AX = 0003h (OwnerCallDvc0Disc)

BX = I14RUserPktID of Device 0 Owner

CL = Host Index (0-15)

CH = Device Address of Detached Device (2-127)

DL = Interface Number that matching Alternate Interface is on

DH = Matching Alternate Interface Number

Notifies all registered device 0 owners whose search criteria

match the characteristics of the detached device (I14RVendorID,

I14RProductID, I14RDvcClass, I14RDvcSubClass, I14RDvcProtocol,

I14RIntfClass, I14RIntfSubClass, and I14RIntfProtocol).

Notes: This request is issued by a host driver as a result of

receiving a "Register As Device 0 (New Interface) Owner"

request (page 123) from another "outside" program.  The first

host driver to receive the initial "outside" request will

assign the device 0 owner handle number (0-based device 0

ownership table index number), put the handle number in

I14RRequestHandle field, and then "pass on" this request to

all host drivers individually.  Each individual host driver

must update its own device 0 ownership table appropriately as

a result of this request.

Each individual host driver must "translate" a 0000h in the

I14RVendorID and/or I14RProductID fields to FFFFh before

storing them in the device 0 ownership table.  FFFFh (not

0000h) must be stored in the device 0 ownership table for

"Don't Care" in the vendor ID and product ID fields, but the

host driver that received the initial "Register As Device 0

(New Interface) Owner" request is not required to perform the

translation before passing on this request.

Each individual host driver must maintain its own copy of the

device 0 ownership table, and each individual table must be

EXACTLY the same as all the others.  The exact format of the

table is discussed under the description of the "Copy The New

Device Ownership Table" request below (page 163).

This request registers an "outside" program to be notified

when an interface (device) is first attached to or detached

from the bus.

Depending on the total number of alternate interfaces a

device has, exactly how the search criteria are defined, as

well as other factors, a device 0 owner may be notified

multiple times for a single device attachment/detachment.

BIOS-LEVEL USB API 0.05            Page 160 of 168

If one (or more) of the search criteria is "Don't Care"

(FFFFh or 0000h in the vendor/product ID, or FFh in the

device/interface class/subclass/protocol), it will ALWAYS

"match" the corresponding field from the actual device.  It

is not possible to specifically look for a 0000h or FFFFh in

the actual vendor/product ID fields, nor is it possible to

specifically look for an FFh (vendor-specific) value in the

actual device/interface class/subclass/protocol fields.

This request will match all alternate interfaces on a device,

even if those alternate interfaces are not actually selected.

BIOS-LEVEL USB API 0.05            Page 161 of 168

InterHost Unregister As Device 0 Owner

��������������������������������������

Class:       InterHost

Return Type: Immediate

Entry:

I14RRequestType   = E4h (I14RRTUnRegDvc0Host)

I14RHostIndex     = Host Index (0-15)

I14RRequestHandle = Device 0 Owner Handle

(Device 0 Ownership Table Index)

Return:

CF = Clear

AX = BX = CX = DX = 0

Notes: This request is issued by a host driver as a result of

receiving an "Unregister As Device 0 (New Interface) Owner"

request (page 127) from another "outside" program.  The first

host driver to receive the outside request (with

I14RRequestType = 65h = I14RRTUnRegIntfOwner, and

I14RHostIndex = 255 = All Hosts) will "translate" the request

into this request (with I14RRequestType = E4h =

I14RRTRegDvc0Host), and then issue the request to each host

driver individually (with a specific host index of 0-15).

Each individual host driver must update its own device 0

ownership table appropriately as a result of this request.

Each individual host driver must maintain its own copy of the

device 0 ownership table, and each individual table must be

EXACTLY the same as all the others.  The exact format of the

table is discussed under the description of the "Copy New

Device Notification Table" request below (page 163).

The device 0 owner handle (the 0-based device 0 ownership

table index) is the value that was returned in the CX

register during the "Register as Device 0 (New Interface)

Owner" request (page 123).

BIOS-LEVEL USB API 0.05            Page 162 of 168

Copy the New Device Ownership Table

�����������������������������������

Class:       InterHost

Return Type: Immediate

Entry:

I14RRequestType = E5h (I14RRTCopyNDTable)

I14RHostIndex   = Host Index (0-15, 255 = Any Host)

(existing Host Index(es))

I14RDataAddress = Address to copy the Table to (Segment:Offset)

I14RDataSize    = 2048 (Number of bytes in the Table)

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Notes: This request is issued by a new host driver during its

initialization process if other host driver(s) were installed

before it was.

This request tells an existing host driver to create a copy

of its new device ownership table at a specific memory

address (I14RDataAddress).  This is how a new host driver

being installed would obtain a copy of the existing table.

I14RDataAddress must be in a segment:offset format.  It

cannot be a 32-bit physical address.  The I14RFlagAddrIsPhys

bit of the I14RFlags field is ignored.

If this request is issued with I14RHostIndex = 255 (any

host), the first existing host driver to receive the request

will respond as if it had received the request with its own

specific host index.  The new host driver can either issue

the request with a specific or non-specific host index and

get the same result.

Each individual host driver must maintain its own copy of the

new device ownership table, and each individual table must be

EXACTLY the same as all the others.  After the table has been

initialized with this request, all host drivers will be

notified of updates to the table with "InterHost Register as

Device 0 Owner" and "Unregister as Device 0 Owner" requests

(pages 159 and 162, respectively).

BIOS-LEVEL USB API 0.05            Page 163 of 168

Here is the exact format of the new device ownership table

(as an A86-compatible structure definition).  Any program

accessing/copying the table must use this exact format.  A

table entry is unused (empty) if the high word (segment

portion) of NDCallBackAddr is 0, regardless of the contents

of any other fields.  The device 0 owner handle (used by

various types of requests) is the 0-based index number of the

table entry (it will always be a number between 0 and 127,

inclusive).

NewDvcNotifyStruc STRUC ;Table of New Device Owners (to be notified

;  when a new Device is attached)

NDVendorID       DW ? ;Vendor ID          |When these are filled

NDProductID      DW ? ;Product ID         |  in, the Call will

NDDvcClass       DB ? ;Device Class       |  only be made if the

NDDvcSubClass    DB ? ;Device SubClass    |  right kind of device

NDDvcProtocol    DB ? ;Device Protocol    |  is attached (not

NDIntfClass      DB ? ;Interface Class    |  just any device)

NDIntfSubClass   DB ? ;Interface SubClass |-1 = "don't care"

NDIntfProtocol   DB ? ;Interface Protocol |

NDUserPktID      DW ? ;User Pkt ID to have in BX during the Call

NDCallBackAddr   DD ? ;Address (CS:IP format) to call when the

;  Device is attached/detached

;0 in Segment here is an Empty Table Entry

ENDS

NDEntries EQU 128                        ;Number of Table Entries

NDSize    EQU (TYPE NewDvcNotifyStruc)   ;Size of each Table Entry

NDTable    DB (NDEntries*NDSize) DUP (0) ;The Table itself

BIOS-LEVEL USB API 0.05            Page 164 of 168

Put Host In Debug Mode

����������������������

Class:       Internal

Return Type: Large

Entry:

I14RRequestType  = FDh (I14RRTHostDebug)

I14RHostIndex    = Host Index (0-15, 255 = All Hosts)

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

AX = CX = DX = 0

BX = I14RUserPktID of Requester

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications (Success):

AX = 001Dh (OwnerCallHostDebug)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = DX = 0

Notifies interface owners of all devices attached to the

host(s), all device 0 owners, and the frame timing owner.

Notes: This request puts the host(s) into debug mode.  This will

stop the host(s) from generating timing on the bus(es), but

allows for single-step requests to be issued which will cause

the bus(es) to increment one frame at a time.  A "Host Run"

request (page 48) should be issued to cause the host to begin

running "normally" again.

BIOS-LEVEL USB API 0.05            Page 165 of 168

If this request is issued with I14RHostIndex = 255 (all

hosts), the first host driver to receive the request will

"pass on" the request to all host drivers individually.

This request may be specific to UHCI hosts.  It is unknown at

this time if hosts other than UHCI even have debug modes, and

if they do, if they are functionally equivalent to the debug

mode for UHCI hosts.

BIOS-LEVEL USB API 0.05            Page 166 of 168

Single Step Host (Debug Mode)

�����������������������������

Class:       Internal

Return Type: Large

Entry:

I14RRequestType  = FEh (I14RRTHostSingleStep)

I14RHostIndex    = Host Index (0-15, 255 = All Hosts)

I14RCallBackAddr = Large Return Address (CS:IP)

I14RUserPktID    = Value to return in BX on Large Return

Return (Success):

CF = Clear

AX = 0

BX = CX = DX = FFFFh

Return (Error):

CF = Set

AX = Error Code (see Error Code Table on page 35)

BX = CX = DX = FFFFh

Large Return (Success):

ZF = Set

AX = CX = DX = 0

BX = I14RUserPktID of Requester

Large Return (Error):

ZF = Clear

AX = 8000h (TDStsLargeCallErr)

BX = I14RUserPktID of Requester

CX = DX = 0

Owner Notifications (Success):

AX = 001Eh (OwnerCallHostSST)

BX = I14RUserPktID of Owner

CL = Host Index (0-15)

CH = DX = 0

Notifies interface owners of all devices attached to the

host(s), all device 0 owners, and the frame timing owner.

Notes: This request single-steps the host(s), after a "Put Host In

Debug Mode" request (page 165) has been issued.  This will

cause the bus(es) to increment the frame index by one.

This request requires that the host be in debug mode before

it is issued.  If the host is not in debug mode, the request

will return with a large call error.

BIOS-LEVEL USB API 0.05            Page 167 of 168

If this request is issued with I14RHostIndex = 255 (all

hosts), the first host driver to receive the request will

"pass on" the request to all host drivers individually.

This request may be specific to UHCI hosts.  It is unknown at

this time if hosts other than UHCI even have debug modes, and

if they do, if they are functionally equivalent to the debug

mode for UHCI hosts.

BIOS-LEVEL USB API 0.05            Page 168 of 168